Rachad ALAO											05/13/98

IEEE 1394 vs USB

USB and IEEE 1394 are 2 bus standards that are shares several characteristics in common, but are somewhat different.  Indeed both of them are serial, hot plug and play buses. They all support both isochronous and asynchronous transfer modes, they uses almost the same kind of cable ( external aspect ). But their ends the comparison.

In fact this two buses target 2 different market of the PC/Consumer Electronic industry : USB targets the low speed peripheral interconnect ( mice, etc. = parallel + I/O ports today )  market where as 1394 targets the high speed peripheral devices ( hard drives, scanner, etc. = SCSI today ). These difference in the target market leads to important differences in the internal specification of these 2 buses :

|  | USB | IEEE 1394 |
| --- | --- | --- |
| Maximum Transfer Rate | 12 Mbits / s | 400 Mbits / s |
| Maximum Number of Device | 127 | 63 per bus and 1023 buses |
| Inter-device max cable length | 5 m | 4.5 m |
| Power supply on Cable | 5V | 8 to 30 V |
| Number of pins per cable | 4 ( 1 twisted data pair, 2 power cables ) | 6 ( 2 twisted data pair; 2 power cables ) |
| Topology | Passive Tree with passive hubs, no leaf can manage the bus. | Active Tree with Active Controller at each node |
| Price Range | Current Serial and Parallel devices price range  ideal for generic PC connectivity | Current SCSI devices price range  ideal for Hard Drive, real time video devices. |