Rachad ALAO ( ralao@email.enst.fr )								06/08/98

IEEE 1394 Link Layer Specifications

Abstract :  The goal of this paper is to describe  the specifications of an architecture of a generic Link Layer for IEEE 1394 buses. The Document will describe the block level architecture, the internal registers available and the different states of the Link state machine.

I . Block Level Architecture :

The Block Level architecture of the IEEE 1394 Link layer described in the IEEE 1394 controller architecture is as follow ( see figure 1 ) :

figure 1.

- Specs of the Phy Interface :

This block interfaces the internal signals from the transmitter, the state machine and the receiver with the Phy Layer. The Block Interface is described in figure 2.

figure 2.

Link_DS : This output indicates that the data on Link_data  and the address on Link_Add are strobe and can be sampled by the Phy Layer.

Link_R/W :  This output indicates whether the Link want to execute a read ( R/W = High ) or a Write ( R/W = Low ) operation.

Link_Add : This 8 bits output specifies the address where the next read or write operation should take place.

Link_Data :  This 8 bits output represents the data to be written in the Phy Layer by the Link layer.

Phy_DS : This input indicates that the data on Phy_Data are strobe and can be  sampled by the Link layer.

Phy_Data : This 8 bits input represents the data to be written in the Link Layer by the Phy Layer.

Receive : This intput indicates the Link layer that data are being received from the bus.

Transmit_Granted :  This intput indicates the Link layer that a previous transmit request has been granted.

Hold : This output indicates the Phy layer that the Link Layer is preparing the data to be sent after the bus was granted and that the Phy layer should try to hold the bus as long as possible.

Link_request : This output indicates the Phy layer that the bus layer want to send data on the bus. A bus request must be sent in the command register of the Phy layer concurrently to the activation of this line.

Clk : This is the Phy/Link clock, it is a 49.152 Mhz Clock as defined in the IEEE 1394 standard.

Link_On : This input indicates the Link Layer that the Link is on the IEEE 1394 cable.

Power_Down : This output indicates the Phy Layer that the device is off.

/Reset :  This output is the general reset output It will reset the Phy layer and may resets the bus.

Transmit_DS : This input pin indicates that the data on Transmit_Data are strobe and must be transmit to the Phy Layer.

Transmit_Data : This 8 bits input pins are the data packet to be sent to the Phy Layer.

Control_R/W : This input indicates if the Link_State_Machine want to perform a read or a write operation on the Phy Layer.

Control_Add  : This 8 bits input contains the Address where the next Read or Write from the Link State Machine must take place.

Control_Dout_DS : This input line indicates that the data from the Link State Machine to be output to Phy are strobe.

Control_Dout : This 8 bits input contains the data from the Link State Machine to be output to Phy.

Control_Din_DS : This output line indicates that the Phy Layer is sending control data and that they are strobe on Control_Din.

Control_Din : This 8 bits output contains the control data sent by the Phy Layer to the Link State Machine.

Receive_DS : This output pin indicates that Phy is sending data packet to the Link Layer ( receive is high )

Receive_Data : This 8 bits output pins are the data packet received from phy.

- Specs of the Link  Transmitter :

This block is responsible for data transmission rate management, CRC encoding and packet framing.

It basically reclocks the data packet and send them at the correct speed to the Phy Layer. It also adds the CRC code at the end of the Header and at the end of the Data section. The Block interface is described in figure 3.

figure 3.

Transmit_DS : This output pin indicates that the data on Transmit_Data are strobe and must be transmit to the Phy Layer.

Transmit_Data : This 8 bits input pins are the data packet to be sent to the Phy Layer.

Transmit_packet : This input indicates that the State Machine is transmitting a packet. Simultaneously to this Speed_code must contain the transmission speed and Frame position must be set to  header_pos.

Speed_code : This 2 bits input contains the speed code of the speed at which the packet will be sent . The different code are the following :

| Speed Code ( binary ) | Effective Speed ( Mbit / s ) |
| --- | --- |
| 00 | 98.304 |
| 01 | 196.608 |
| 10 | 393.216 |
| 11 | 786.432* |

- *not implemented yet

Frame_pos : This 2 bits input indicates which part of the packet is being transmitted at the moment. The encoding of this bits are the following :

| Frame_pos ( binary ) | Meaning |
| --- | --- |
| 00 | Header |
| 01 | Data |
| 10 | Reserved |
| 11 | EOF |

Data : The 32 bits data to be added to the frame.

Next_data_quad : This output indicates the Link State Machine that it must send the next 32 bits of data at the next rising edge of the clock.

CRC_data_out : This output indicates to the CRC module that the next 32 bits of CRC_data are strobe. It must be maintained high for one clock cycle.

CRC_generate : This output indicates the CRC module that it must generate a 32 bits CRC pattern.

CRC_data : This 32 bits output contains the data sent to the CRC module.

CRC_pattern : This 32 bits input contains the CRC pattern generated by the CRC module when CRC_pattern_ready pin is high.

CRC_pattern_ready : This input indicates the transmitter that the CRC pattern has been generated and is strobe on CRC_pattern pin.

- Specs of the CRC Module :

This module knows how to generate IEEE 1394's 32 bit CRC for the Header and for the Data section. It generates this 32 CRC incrementally until CRC_generate is activated. Then the CRC_pattern is output and CRC_ pattern ready is set high until the CRC_data_in is asserted high. The Block interface is described in figure 4.

figure 4.

CRC_data_in : This input indicates to the CRC module that the next 32 bits of CRC_data are strobe. It must be maintained high for one clock cycle.

CRC_generate : This input indicates the CRC module that it must generate a 32 bits CRC pattern.

CRC_data : This 32 bits input contains the data sent to the CRC module.

CRC_pattern : This 32 bits output contains the CRC pattern generated by the CRC module when CRC_pattern_ready pin is high.

CRC_pattern_ready : This input indicates the transmitter that the CRC pattern has been generated and is strobe on CRC_pattern pin.

- Specs of the Link Receiver :

This module is responsible for data reception rate management, CRC checking and data filtering ( by checking the Destination ID ). The Block interface is described in figure 5.

figure 5.

Receive_DS : This input pin indicates that Phy is sending data packet to the Link Layer.

Receive_Data : This 8 bits input pins contains the data packet received from phy.

Receive : When asserted high this output pin indicates the Link State Machine that a data packet is being receive.

Data_quad_strobe : When asserted high this output pin indicates the Link State Machine that it must read the a data quadlet at the next rising edge of the clock.

Data_quad : This 32 bits output pins contains the data quad being received from Phy when Data_quad_strobe is asserted high.

Physical_ID : This 16 bits input pin contains the Unique Physical ID of the device : ( Bus_ID[15..6], Node_ID[5..0] ).

Check_CRC : When asserted high this input pin indicates the receiver that it must start checking the CRC of the block previously received before the next rising edge of the clock.

CRC_check_done :  This output pin indicates the Link State Machine that the CRC check has been done and that the result is available on the CRC_correct pin.

CRC_correct : This output pin indicates the result of the last CRC check. When asserted high it indicates that the last CRC check was correct.

CRC_data_out : This output indicates to the CRC module that the next 32 bits of CRC_data are strobe. It must be maintained high for one clock cycle.

CRC_generate : This output indicates the CRC module that it must generate a 32 bits CRC pattern.

CRC_data : This 32 bits output contains the data sent to the CRC module.

CRC_pattern : This 32 bits input contains the CRC pattern generated by the CRC module when CRC_pattern_ready pin is high.

CRC_pattern_ready : This input indicates the transmitter that the CRC pattern has been generated and is strobe on CRC_pattern pin.

- Specs of the Isochronious Manager :

This module holds the cycle timer register as defined in IEEE 1394 specifications. This register can be updated by cycle_start packet and incremented by the local 24.59 Mhz clock or it can be entirely managed by the local node if node is isochronious cycle master. The Block interface is described in figure 6.

figure 6.

Cycle_master : When asserted high this input pin indicates that the node is cycle master and must generate the cycle start packet of the IEEE 1394 bus.

Update : When asserted high it indicates that the Cycle Timer register of the isochronious manager must be updated with the Cycle Start packet available on the Cycle_start_quad 32 bits pin.

Cycle_start_quad : this 32 bits pin contains the cycle start quadlet when the Update pin is asserted high.

Cycle_timer_quad : This 32 bits output pin contains the Cycle Timer register value permanently.

Send : This output pin indicates the Link State Machine that a new cycle start must be sent on the bus. It is used only if the node is Isochronious cycle master.

- Specs of the Isochronious Monitor :

This module monitors the isochronious cycles on the bus and plans the isochronious transmissions according to the Isochronious Transmission FIFO state. The Block interface is described in figure 7.

figure 7.

Cycle_timer_quad : This 32 bits input pin contains the value of the cycle timer register from the isochronious manager.

Channel_reserved : This 64 bits input pins indicates the channels officially reserved by this node.

Isoch_bus_req : This output pin indicates the Link Layer State Machine that it should request the bus to send the isochronious packet pending on channel Isoch_channel.

Isoch_channel :  This 6 bits output pin indicates the ID of the channel requested.

- Specs of the Link Layer State Machine :

This module is the heart of the Link Layer, It holds all the registers defined in IEEE 1394 standard, it knows how to handle data packets, how to answer transmission requests from the FIFO Controller. The different states of this machine will not be defined in this paper. For further description please refer to the VHDL code on the link state machine. The block interface of this state machine is described in figure 8.

figure 8.

Control_R/W : This output indicates if the Link_State_Machine want to perform a read or a write operation on the Phy Layer.

Control_Add  : This 8 bits output contains the Address where the next Read or Write from the Link State Machine must take place.

Control_Dout_DS : This output line indicates that the data from the Link State Machine to be output to Phy are strobe.

Control_Dout : This 8 bits output contains the data from the Link State Machine to be output to Phy.

Control_Din_DS : This input line indicates that the Phy Layer is sending control data and that they are strobe on Control_Din.

Control_Din : This 8 bits input contains the control data sent by the Phy Layer to the Link State Machine.

Transmit_Granted :  This intput indicates the Link layer that a previous transmit request has been granted.

Hold : This output indicates the Phy layer that the Link Layer is preparing the data to be sent after the bus was granted and that the Phy layer should try to hold the bus as long as possible.

Link_request : This output indicates the Phy layer that the bus layer want to send data on the bus. A bus request must be sent in the command register of the Phy layer concurrently to the activation of this line.

Transmit_packet : This output indicates that the State Machine is transmitting a packet. Simultaneously to this Speed_code must contain the transmission speed and Frame pos must be set tot header_pos.

Speed_code : This 2 bits output contains the speed code of the speed at which the packet will be sent by the transmitter . The different code are the following :

| Speed Code ( binary ) | Effective Speed ( Mbit / s ) |
| --- | --- |
| 00 | 98.304 |
| 01 | 196.608 |
| 10 | 393.216 |
| 11 | 786.432* |

- *not implemented yet

Frame_pos : This 2 bits input indicates which part of the packet is being transmitted at the moment by the State Machine to the transmitter. The encoding of this bits are the following :

| Frame_pos ( binary ) | Meaning |
| --- | --- |
| 00 | Header |
| 01 | Data |
| 10 | Reserved |
| 11 | EOF |

Transmit_data : The 32 bits data to be added to the frame by the transmitter.

Transmit_next_data_quad : This input indicates the Link State Machine that it must send the next 32 bits of data to the transmitter at the next rising edge of the clock.

Receive : When asserted high this input pin indicates the Link State Machine that a data packet is being receive.

Data_quad_strobe : When asserted high this input pin indicates the Link State Machine that it must read the a data quadlet at the next rising edge of the clock.

Data_quad : This 32 bits input pins contains the data quad being received from Phy when Data_quad_strobe is asserted high.

Physical_ID : This 16 bits output pin contains the Unique Physical ID of the device : ( Bus_ID[15..6], Node_ID[5..0] ).

Check_CRC : When asserted high this output pin indicates the receiver that it must start checking the CRC of the block previously received before the next rising edge of the clock.

CRC_check_done :  This input pin indicates the Link State Machine that the Receive CRC check has been done and that the result is available on the CRC_correct pin.

CRC_correct : This input pin indicates the result of the last Receive CRC check. When asserted high it indicates that the last Receive CRC check was correct.

FIFO_DS : This input indicates that the data on FIFO_data  and the address on FIFO_Add are strobe and can be sampled by the Link Layer.

FIFO_R/W :  This input indicates whether the FIFO Controller want to execute a read ( R/W = High ) or a Write ( R/W = Low ) operation.

FIFO_Add : This 8 bits input specifies the address where the next read or write operation should take place in Link Layer.

FIFO_Data :  This 32 bits input represents the data to be written in the Link Layer by the FIFO Controller.

Link_DS : This output indicates that the data on Link_data  and the address on Link_Add are strobe and can be sampled by the FIFO Controller.

Link_R/W :  This output indicates whether the Link want to execute a read ( R/W = High ) or a Write ( R/W = Low ) operation .

Link_Add : This 3 bits output specifies the address where the next read or write operation should take place in the FIFO Controller.

Link_Data :  This 32 bits output represents the data to be written in the FIFO Controller by the Link layer.

This State Machine must also hold in addition to its operation registers the registers defined in the IEEE 1394 standards.  These registers can be accessed through the IEEE 1394 bus at the base address FFFF F000 0000 h. The list of these registers is as follow :

Register Name : NODE_IDS			Offset : 008 h

Usage : This register holds the current bus ID and current physical ID of this node.

| Bus_id ( 10 bits ) | Phy_id ( 6 bits ) | Zeros ( 16 bits ) |
| --- | --- | --- |

Register Name : RESET_START			Offset : 00C h

Usage : A write to this register performs an immediate reset.

Register Name : SPLIT_TIMEOUT		Offset : 018 h

Usage : Specifies the maximum time a node waits for a response subaction after a request subaction. The transaction is aborted after this time.

| Zeros ( 29 bits ) |  | Sec ( 3 bits ) |
| --- | --- | --- |
| Fraction of second in 125us ticks ( 13 bits ) | Zeros ( 19 bits ) |  |

Register Name : CYCLE_TIME			Offset : 200 h

Usage : This register is nothing but the Cycle time register hold by the Isochronious manager of the node.

| Sec_count ( 7 bits ) | Cycle_count ( 13 bits ) in 125us ticks | Cycle_offset ( 12 bits ) in 25.576 Mhz clock ticks |
| --- | --- | --- |

Register Name : BUS_TIME			Offset : 204 h

Usage :  Holds a measure of the current time in second and does not reinitialize after a bus reset.

| Second_count_hi ( 25 bits ) | Second_count_low |
| --- | --- |

Register Name : BUSY_TIMEOUT		Offset : 210 h

Usage : Is used to control the number of retries. The maximum number of retries is given by retry_limit.

| Reserved ( 4bits ) | S_lim | Cycle_limit ( 13 bits ) | Reserved ( 8 bits ) | Retry_limit ( 4 bits ) |
| --- | --- | --- | --- | --- |

Register Name : BUS_MANAGER_ID		Offset : 21C h

Usage :  Gives the physical ID of the current bus manager. The initial value of this field is 3F h .

| Reserved ( 26 bits ) | Bus_mngr_id |
| --- | --- |

Register Name : BANDWIDTH_AVAILABLE	Offset : 220 h

Usage :  Holds the bandwidth remaining that can be allocated for isochronious transfer, it is expressed in unit equal to the time need to transfer a data quadlet at S1600 speed. The initial value of Bw_remaining is 4915 .

| Reserved ( 19 bits ) | Bw_remaining ( 13 bits ) |
| --- | --- |

Register Name : CHANNELS_AVAILABLE		Offset : 224 h

Usage : Indicates the channels that are currently used for isochronious transfer. A zero at a given position indicates that the corresponding channel is allocated.

Channels_available_hi ( 32 bits )

Channels_available_low ( 32 bits )

Register Name : TOPOLOGY_MAP			Offset : 800 h

Usage : This register is the registration of the self_ID packets observed. This map may be used to optimize the gap_count value. The length field specifies the number of following quadlets. The CRC field covers all the following quadlets. The node_count field gives the number of nodes present on the local bus. The  self_id_count field gives the number of self_ID packets following.

| Length ( 16 bits ) | CRC ( 16 bits ) |
| --- | --- |

Generation_number

| Node_count ( 16 bits ) | Self_id_count ( 16 bits ) |
| --- | --- |

Self_ID packet [ 0.. Self_id_count - 1] ( 32 bits )