# Acknowledgements

- Before all talks I would like to thank all the people that helped me in the realization of this "mémoire". I would like to thank specially the persons named bellow for the appreciable help they gave me at the beginning and all along the elaboration of this report.

- Jean Luc DANGER, Professor, Department of Electrical Engineering, "Ecole Nationale Supérieure des Télécommunications de Paris".

- Ulrich FINGER, Director of the Computer Science and Network Department, "Ecole Nationale Supérieure des Télécommunications de Paris".

- Renaud PACALET, Professor, Department of Electrical Engineering, "Ecole Nationale Supérieure des Télécommunications de Paris".

# Table of Content

Acknowledgements	2

Table of Content	3

Introduction	5

A.	Overview of the IEEE 1394 bus	6

I. General Overview :	6

1. Topology :	7

2. Type of Transactions :	8

3. Protocol's Structure :	9

II. The Physical Layer :	10

1. The connection and the physical signals :	10

2. The Physical packets :	13

3. Gaps :	13

4. Initialization :	14

5. Arbitration :	15

III. Bus Management Layer :	18

1. CYCLE MASTER :	19

2. ISOCHRONOUS MANAGER :	20

3. BUS MANAGER :	20

4. Determination of the ISOCHRONOUS MANAGER :	21

5. Determination of the BUS MANAGER :	21

6. Determination of the CYCLE MASTER :	22

IV. Data Packet Delivery :	22

1. Asynchronous packets :	22

2. Isochronous packets :	24

3. Acknowledgement packets :	25

V. Transactions :	25

1. Transaction operation :	25

2. Types of transaction :	26

VI. Example of Data Transfer :	26

1. Example of Asynchronous Transfer :	27

2. Example of Isochronous Transfer :	28

VII. USB versus 1394 :	29

B. Hardware Architecture of a generic IEEE 1394 Solution	31

I. Global Project Overview :	31

II. IEEE 1394 Controller functional block overview :	32

III. Block Level Detailed Architecture :	32

1. Phy Layer :	32

2. Link Layer :	33

3. FIFO Controller :	34

4. Local Host Bus Adapter :	35

5. Software Component :	35

IV. Physical Layer Specification :	35

1. Specs of the Cable Analog Interface :	35

2. Specs of the Transmit Encoder Block :	36

3. Specs of the Receive Decoder Block :	38

4. Specs of the Phy State Machine :	39

V. Link Layer Specification :	41

1. Specs of the Phy Interface :	42

2. Specs of the Link  Transmitter :	44

3. Specs of the CRC Module :	46

4. Specs of the Link Receiver :	47

5. Specs of the Isochronous Manager :	48

6. Specs of the Isochronous Monitor :	49

7. Specs of the Link Layer State Machine :	49

C. Validation Method : My 1394 Simulator	55

I. 1394 Software Simulator :	55

1. Software controller :	55

2. 1394 Simulator :	57

II. 1394 Checker :	57

Conclusion	59

Annexes	60

I. Tree ID Process :	60

II. 1394_Phy.h :	63

III. 1394_Link.h :	66

IV. 1394_Trans.h :	70

V. 1394_Controller.h :	72

VI. 1394_Network.h :	74

References	76

# Introduction

We are nowadays leaving the last hours of  the old good Parallel, Serial and SCSI ports of our PCs and workstations. Indeed they appeared decades ago and are still in used today in most PCs; most peripherals sold currently are still using either a Parallel port, a Serial Port or a SCSI bus  to access the host computer. The success of these buses was due mainly to the simplicity of implementation of the peripheral controllers ( except for the SCSI bus ) between the host computer and the peripheral and to the availability of cheap integrated solutions for this type of connection.  But nowadays new applications require several services which were not provided by any of these ports : For instance large bandwidth, maximum jitter guaranty, bandwidth guaranty, hot plug and play, I/O integration ( to limit PCB space used and number of software needed ), etc.

Thus, several solutions were proposed to address these critical issues but only 2 of them were both standardized by the IEEE and strongly recommended by the PC consortium. They are named Universal Serial Bus ( USB ) and IEEE 1394 bus ( known as Firewire® which is a trademark  of Apple Computer ) . They are both serial buses, hot plug and play, with isochronous abilities ( ability to handle real time transfers ), but they target different applications, since USB targets low to medium bandwidth ( < 12 Mb/s ) applications and IEEE 1394 targets high bandwidth applications ( up to 800 Mb/s ). The differences between these two buses will be illustrated in another part of this document

The goals of this report are multiple : First we want to lead the reader to a complete understanding of IEEE 1394 high-speed serial bus standard, second we want to give some detailed specifications of the architecture of a IEEE 1394 solution for the designer who would like to design his own controller, and last we want to give a validation approach to check this kind of controller by introducing a IEEE 1394 bus simulator.

In order to correctly deal with these main focuses, these report will start by a general overview of the IEEE 1394 standard, then we will introduce a hardware architecture of a generic IEEE 1394 solution and we will end by the presentation of the IEEE 1394 Simulator.

# Overview of the IEEE 1394 bus

## I. General Overview :

The IEEE 1394 standard defines a new serial bus for high speed I/Os which goals are multiple.

First it must be a low cost, high performance ergonomic peripheral bus, based on Read/Write memory architecture ( somewhat like SCSI ). Second it must be compatible with other serial buses and handle Isochronous service ( meaning bandwidth guaranty, jitter control, etc. ). Examples of isochronous applications are Audio application like CD running at 44.1 kHz * 16 bits * 2 channels = 1.4 Mbps, or Video application like MPEG2 or raw high definition video which need a guaranteed bandwidth varying between 1.5 and 216 Mbps depending on the type of compression and on the number of frames per second transmitted. The main characteristics of Asynchronous transport and Isochronous transport are summarized in the board bellow.

| Asynchronous Transport | Isochronous Transport |
| --- | --- |
| - Guaranteed delivery ( CRC + retry on failure ) - The reliability of the transport is more important than the timing - Retry are accepted | - Guaranteed timing ( jitter and bandwidth are guaranteed ) - Late data are useless - Therefore retry are useless |

The IEEE 1394 must also be an  unsupervised bus with automatic address selection and arbitrary location, contrarily to the SCSI bus and as shown on figure 1 and figure 2.

figure 1.

figure 2.

### 1. Topology :

The IEEE 1394 standard defines a new serial bus for high speed networks. The speed supported by the standard are 100, 200, 400 and 800 Mbit/s. The topology of a IEEE 1394 serial bus is described in figure 3.

figure 3.

The cable topology is a non-cyclic network with finite branches. Non-cyclic means that you cannot connect together nodes so as to create a loop. Up to 16 cable portions are allowed between two nodes and each cable portion can be as long as 4.5 meters.

Each node has a 16 bits address, called node_ID, that allows space for up to 64K nodes. This node_ID is divided into two fields : The 10 first bits specify the bus address and the next 6 bits specify the physical_ID. These two fields reserve the value of all "1" for special purpose, so that you can only have 1023 buses and 63 nodes on each bus.

- Node_ID[ 15 .. 0 ] = Bus_ID[ 15 .. 6 ] || Physical_ID[ 5 .. 0]

Each bus is separated from other buses by a bridge. A bridge eavesdrops on the bus, ignoring all transactions on local bus but listening carefully for remote transactions. When a bus receives data for a remote node, it forwards this data to an adjacent bus. Each node can be connected to up to 27 ports.

### 2. Type of Transactions :

As said earlier the 1394 standard uses two types of transfers : asynchronous and isochronous transfers. The process of delivering a packet is called in the IEEE 1394 standard a subaction. Here are listed the different type of subaction defined in the standard :

- An asynchronous subaction is a transfer of a variable amount of data to a specified address and an acknowledgement is returned.

- An asynchronous broadcast subaction is a transfer of a variable amount of data to a non specified address and no acknowledgement is returned.0

- An isochronous subaction is a real-time transfer of a variable amount of data to a specified "channel" at a regular intervals. No acknowledgement is returned.

A subaction can have three parts :

- An arbitration sequence : A node that wishes to transmit a packet requests the control of the bus. If several nodes are requesting this control at the same time, the network uses an arbitration process to determine which of this nodes will gain the control.

- A data packet : Contains the information to be transferred. The packet begins with a data_prefix and ends with either a data_prefix or data_end.

- An acknowledgement : This is a small packet sent in response to an asynchronous packet.

In order to save time and in particular circumstances ( i.e. when responding node wants to send data

just after its acknowledgement or when a node wants to send several isochronous packets ) the subaction can be concatenated. Then instead of ending by a data_end the data packet ends by the data_prefix of the next subaction to be sent. The figure 4 shows the typical structure of a data packet over the IEEE 1394 bus.

figure 4.

### 3. Protocol's Structure :

The structure of the IEEE 1394 protocol can be described as a three layer architecture as shown in

figure 5. This architecture is  somewhat similar to the three lowest levels of the OSI model of the ISO.

figure 5.

- The Physical Layer : The physical layer called PHY in the standard is responsible for power-up initialization, arbitration, bus reset sensing and data signaling.

- The Link Layer : The link layer  is responsible for addressing, data checking and data framing for packet transmission.

- The Transaction layer : The transaction layer defines a complete request-response protocol to perform bus transactions. This Layer will be in our implementation mostly a software emulation to reduce hardware complexity.

The different packets are gathered into groups called cycles. Each cycle is about 125 us long and is

shared between isochronous and asynchronous transmission.

The bus management consists of the functions that control the formation of the cycles and the data

flow between the different layers.

The  node responsible for the generation of these cycles is the CYCLE_MASTER.

The node responsible for the control of isochronous transmission is the ISOCHRONOUS MANAGER.

The node responsible for the rest of the bus management is the BUS MANAGER.

## II. The Physical Layer :

### 1. The connection and the physical signals :

The Cable's connection :

The cable consists of three pairs of wires. Two of these pairs are shielded low-voltage differential

pairs that are used to transmit the data. The last pair is a power pair that allows the PHY layer to stay powered even if the application is switched off. The cable is described on figure 6.

figure 6.

The two devices are connected so that the pair A of one device is connected to the pair B of the other device as shown on figure 7.

figure 7.

Signal characteristics :

The physical signals used over the bus can take three different states depending on the differential

amplitude read on the pairs.

The data rate of the cable media is 98.304 Mb/s  minimum. This rate is called the base rate. Ports can support higher rates 196.608 Mb/s 393.216 Mb/s and recently 786.432 Mb/s. These rates are referred to as S100, S200 and S400 speeds. But all ports supporting higher rates must support lower rates. When a connection is to be established, the cable can be in two different phases. One is the data transmission phase, the other is the arbitration phase.

Data transmission signals :

When transmitting data, the bus communicates via NRZ data, so only the state 1 and 0 are used.

The pair B transmits the data while the pair A transmits the strobe. The strobe signal changes state when two consecutive NRZ data bits are the same. Thus a clock that transitions every bit period can be extracted by making an XOR of data and strobe ( see figure 8 ).

figure 8.

Arbitration phase signals :

This phase uses the  three cables states 1, Z, 0. Then in the arbitration phase, the physical layer

uses decoding rules that respect the one's dominance rule since collision may happened in this phase. These rules are given in the table bellow :

| Received Signal | Transmitted Signal | Interpreted Signal | Comment |
| --- | --- | --- | --- |
| Z | Z | Z | If the port is transmitting Z the received signal is the same than the transmitted signal of the other port |
| 0 | Z | 0 | The same rule is applied |
| 1 | Z | 1 | The same rule is applied |
| Z | 0 | 1 | If receiving Z while transmitting 0 the other port is transmitting a 1 |
| 0 | 0 | 0 | The other port is transmitting either 0 or Z |
| Z | 1 | 0 | If receiving Z while transmitting 1, the other port is transmitting 0 |
| 1 | 1 | 1 | The other port is transmitting either 1 or Z |

The combination of the state of the two signal pairs gives a set of physical layer states that are used during the arbitration. These states are described in the second part of this report ( B. Hardware architecture, IV. Physical Layer Specification ).

### 2. The Physical packets :

The physical layer uses different packets for the bus management. These packets consists of 64

bits, the second 32 bits being the logical invert of the first 32 bits ( more error proof than CRC but more bit expensive ). If the last 32 bits do not match with the first 32 bits, the packet is ignored by the physical layer. They are all at the S100 speed.

Self _ID packet :

There can be one to four self_ID packets sent by the node during its self_ID phase of arbitration.

This packet gives information about the sending node:

- The physical_ID.

- The fact that its link layer is active or not .

- The speed capabilities of the node.

- The fact that the node is Bus and Isochronous Manager capable;

- The power consumption class.

- The number of ports and their status :

- not connected;

- connected to child node

- connected to parent node

- Initiate a bus reset.

Link_On packet :

This packet is sent for these nodes that do not automatically power on their link layer. It forces the power on of the link layer of the destination packet.

PHY configuration packet :

This packet is sent to optimize the serial bus performances. It sets the gap_count value ( a value

used to optimize gap delays ) and it can force one node to become the root.

### 3. Gaps :

A gap is a period between two packets during which the bus is idle ( the two signal pairs are in

State Z ). There are four types of gaps :

- Isochronous gap : gap between two isochronous packets.

- Acknowledge gap : gap between an asynchronous packet and its acknowledgement.

- Subaction gap : gap between an acknowledgement and a new asynchronous packet.

- Arbitration gap : gap between two fairness intervals.

The values of these gaps are gap-count field dependent and are given in annexes.

### 4. Initialization :

The initialization is divided into three phases :

- The bus reset

- The tree identification

- The self-identification

All these processes use the arbitration signals defined in the IEEE 1394 standard.

Bus Reset :

A bus reset occurs if one of the following happen :

- The serial bus management makes a PHY configuration request, this request is done by a self_ID packet

- The PHY detects a change in any port's connection state.

- Th PHY stays in any state other than idle, reset_wait, transmit or receive for longer than the maximum time allowed for arbitration MAX_ARB_STATE_TIME.

- The PHY receives a BUS_RESET ( state 11) on any ports.

Then the node send a BUS_RESET for a time long enough, i.e. for RESET_TIME. After this time

the node sends idle state and waits for answer of the other ports directly connected.

Then if the time to receive idle (ZZ) is greater than RESET_TIME, then the node restarts the reset process. Otherwise if the time to receive idle is lower than RESET_TIME, then the tree identification process starts.

Tree Identification :

This process transforms the general network topology into a tree. One node is designated as the

Root. The ports are given a label that indicates their position. A parent port is connected to a node closer to the root while the child port is connected to a port further from the root. The different steps of the tree_ID process are described in annexes.

Self Identification :

This process gives to each node the opportunity to select a unique physical_ID. At that point the

root has been determined. The Self identification process is done with a deep first exploration algorithm which is detailed bellow.

- Initial state : At the beginning of this process the self_ID_counter of each node, that can be found in the TOPOLOGY_MAP has the value zero. The network is found in the state it had at the end of the tree identification process.

- Start of the self identification process : The root sends a grant ( Z0 )to its lowest numbered unidentified port and data_prefix ( 01 )to all other ports connected.

- Rule 1 : When a node receives a grant from its parent and all its children are still not identified, it transmits a grant to its lowest numbered unidentified port and data_prefix ( 01 ) to all other ports connected.

- Rule 2 : When a node receives a grant from its parent and all its children are identified, it takes the self_ID_count value as its node address ( the phy_ID ).

- The node sends a data_prefix, then self_ID packets and finally a data_end to all its port.

- It then sends idle to its children ( that are identified ) and ident_done to its parent.

- Rule 3 : A node receiving a data_prefix in response to a grant withdraws its grant.

- Rule 4 : A node receiving  the data_prefix, self_ID packets, data_end and idle ( if there is idle ) sequence must transmit to all other nodes connected.

- Rule 5 : A node receiving ident_done must respond by data_prefix to the sender and send idle to all other node connected. The port where it receives this ident_done is marked as identified.

- Rule 6 : When a node receives self_ID ( and data_end ), it increments its self_ID count.

- Rule 7 : The port receiving the data_prefix in response to its ident_done goes idle.

- Rule 8 : When the root is idle on all its ports or see data_prefix on one port and idle on all the others it goes to the start of the self identification process.

After this process, each node has a unique physical node number, and the topology has been broadcast.

### 5. Arbitration :

After the initialization, the network can start the exchange of information. To transmit this information to other nodes, the transmitter node must gain control of the bus. As several nodes may ask the control of the bus at the same time, there has to be a process that determines which of the requesting nodes will be given control of the bus. This process is called arbitration.

In the IEEE 1394 standard, there are four different types of arbitration. Each one being used in particular circumstances :

- Normal arbitration used when sending isochronous packets.

- Fairness arbitration used when sending asynchronous packets.

- Immediate arbitration used when sending an acknowledgement in response to an asynchronous packet.

- CYCLE_MASTER arbitration used when a cycle_synch event is queued to start isochronous transfers.

The different types of arbitration are detailed in the remaining of this chapter.

Normal Arbitration :

This arbitration is used for the transmission of isochronous packets. When an isochronous gap is detected ( remind that the gap is a short period of time where the bus is idle ), all the nodes that want to send isochronous packets can start normal arbitration. The normal arbitration respects the following rules :

- Rule 1 : Nodes begin arbitrating by sending a request ( Z0 ) to its parent and a data_prefix to its children.

- Rule 2 : The parent forwards the request to its parent and denies access to its other children by sending them a data_prefix. The root does not have to forward the request any further.

- Rule 3 : The root grant ( Z0 ) access to the first request and denies access to the other ports ( with a data prefix ).

- Rule 4 : When access is denied, the children acknowledge this denial by withdrawing its request and denying ( with a data_prefix ) the access to children if any was requesting the control of the bus.

- Rule 5 : When a node receives a grant it passes it to the concerned child or it keeps it if it is the node that is arbitrating.

- Rule 6: When the node that was arbitrating receives the grant it responds by sending a data_prefix.

- Rule 7 : When a node receives a data_prefix in response to a grant, it withdraws its grant and transmit the data_prefix to its parent. The root does not have to transmit this data_prefix any further. At that moment the transmission can start because all the node are seeing  a data_prefix from the node that has won arbitration.

The normal arbitration gives the control of the bus to the closest node to the root. If two nodes that are at the same distance from the root ask for the control of the bus, the one with the highest node_Id will be given the control of the bus.

Fairness Arbitration :

As just seen, the closest node to the root is always given control of the bus. So to avoid that a node monopolizes the bus during asynchronous transfer, the fairness arbitration protocol is used. This protocol splits the access opportunity evenly among all the competing nodes.

A fairness interval is the succession of one or several periods of bus activity separated by short idle periods ( subaction gap ) and followed by a longer idle period ( arbitration reset gap ) as shown in figure 9.

figure 9.

A node can initiate sending asynchronous packets only once each fairness interval. Indeed, an active node can arbitrate ( Normal Arbitration ) if  arb_enable signal is set ( this flag is one of the node's variable). This flag is set by an arbitration reset gap and is cleared when the node wins arbitration.

The fairness interval ends when arbitration by the final fair node is successful. This generates an arbitration reset gap since all nodes now have their arb_enable signal reset and cannot drive the bus.  Figure 10 illustrates how arb_enable flag works.

figure 10.

Immediate Arbitration :

When an asynchronous packet is sent it has to be acknowledged, a node receiving an asynchronous packet requests the control of the bus while receiving to be able to acknowledge the packet. This is done by an immediate arbitration which is always granted.

CYCLE MASTER Arbitration :

This arbitration should only be initiated by the root. This occurs when the root is queuing a cycle_synch_event that means that the asynchronous transfer should stop and the isochronous should start. At the next subaction gap the root will be granted the control of the bus even if nodes are requesting this control for asynchronous transfer. Then the root will sends a cycle start packet to indicate to all the nodes that the isochronous transfers are allowed.

After the basics of the Physical layer, we will then detail the  higher layers in the remaining of this report.

## III. Bus Management Layer :

To understand the need of the different bus management functions of the IEEE 1394, one should understand the way the data transmissions function correctly.

The transmission of data is cyclic. A cycle should be 125 s long. On these 125 s, 100 s ( maximum ) are dedicated for isochronous transfers and 25 s (minimum ) for asynchronous transfers. A cycle always begins with a cycle_start packet. This packet indicates that isochronous data can be sent. Then after a 100 s of isochronous transfer, asynchronous data can be sent during 25 s. A cycle_synch_event occurs 125 s after the beginning of the cycle_start packet and indicates that a new cycle should start. However, as the transfer of asynchronous packets might not be finished, the transmission of the cycle start packet might be delayed ( waiting for the end of asynchronous packet ). As this delay would create significant jitter, this delay is communicated with the cycle_start packet. The IEEE 1394 bus cycle is illustrated on figure 11.

figure 11.

The IEEE 1394 standard defines several levels of management that ensure the transmission of data and its cyclic character. These managers are the CYCLE MASTER, the ISOCHRONOUS MANAGER and the BUS MANAGER. There are no requirements on the localization of these managers, they can all be performed by the same node or by different nodes. But there must only be one of each manager. All the variables and registers needed for this management are described in the second part of this report ( B. Hardware architecture, V. Link Layer specifications ).

### 1. CYCLE MASTER :

The function of this manager is to maintain a common clock source. This clock source starts at zero at the beginning of the cycle, and when 125 s are reached, the cycle master queues a cycle_synch_event. As the bus might be controlled by another node transmitting asynchronous data, the cycle master waits for the end of this transmission and the bus being idle for a subaction gap before sending the cycle start packet to other nodes. At the beginning of the transmission of this packet the clock source restarts at zero. The cycle start packet indicates to the other nodes that they can arbitrate in order to send isochronous data.

In the cable environment specifications, it is required that the cycle master is the root ( in order to be sure that it will win arbitration  at the next subaction gap ).

### 2. ISOCHRONOUS MANAGER :

Each cycle, an isochronous application needs to transmit a fixed amount of isochronous data. So, this application must have reserved enough bandwidth during the cycle. The node that handles this function is the isochronous manager.

The procedure for isochronous transmission is described bellow :

- In order to save time and bandwidth usage, and because the source might send the same data to several destination nodes, the sender does not use the node_ID as destination address but a channel that had been allocated by the isochronous manager. The nodes that are supposed to receive this data only have to recognize that channel number and listen to it.

- There must be only one node sending data on a specified channel but several nodes can listen at a same channel.

- One node can send or listen data on different channel.

- The maximum number of channels that can be allocated is 64.

Therefore the Isochronous manager must implement the following functions :

- Isochronous bandwidth allocation.

- Channel allocation.

- Selection of the cycle master ( if no bus manager is present ).

### 3. BUS MANAGER :

The presence of the bus manager is optional. When there is no bus manager, some of the functions that should be handled by the bus manager are handled by the isochronous manager.

The bus manger provides serial bus management services for other nodes such as :

- Selection of a cycle master.

- Power management.

- Maintenance of the topology map and speed map ( maps that describe the topology of the network and the speed of all the nodes connected ).

- Bus optimization based on information obtained from these maps.

### 4. Determination of the ISOCHRONOUS MANAGER :

The selection of the isochronous manager is done during the self identification process. When transmitting its self_ID packet during this process, a node indicates whether or not it is candidate for being the isochronous manager.

The candidate node with the largest phy_ID wins the role of isochronous manager. The node that are candidate to this role should also monitor and check all received self_ID packets to ensure that :

- The second quadlet of each self_ID packet is the logical inverse of the first quadlet.

- The phy_ID field of all first self_ID packets of a node are in monotonically increasing sequence. And the phy_ID field of all self_ID packets other than the first one is the same than the last first self_ID packet.

- The last self_ID packets indicate that all the connected ports are children ( this is the self_ID packets of the root ).

### 5. Determination of the BUS MANAGER :

If a node is bus manager capable, it sends a lock request to the isochronous manager. This request contains two parameters, one is called arg_value and is 3Fh, the other is called data_value and is the phy_ID of the node.

When it receives this request, the isochronous manager compare the arg_value with its BUS_MANAGER_ID register. If the two values are equal ( i.e. both are 3Fh ), the BUS_MANAGER_ID register changes and takes the value of the data_value received ( phy_ID of the sending node ). If the two values are different, the BUS_MANAGER_ID register remains unchanged ( this process is called compare and swap ).

The isochronous manager then sends a lock response to the requesting node. This response contains the value of the BUS_MANAGER_ID register.

When the requesting node receives the response, it compares the BUS_MANAGER_ID received with the data_value it has sent. If they are equal , that means the node is the bus manager, otherwise this field contains the phy_ID of the new bus manager.

So, since the value of the BUS_MANAGER_ID register after initialization is 3Fh, the bus manager capable node closest to the root will become the bus manager because it will win arbitration for lock request and will be the first to send it.

But the process that determines the bus manager must promote the stability of the serial bus management. In this purpose, if there was a bus manager before the bus reset, this bus manager is incumbent and start sending its request as soon as it san after the self identification process. The nodes that are not incumbent will try to send their request 125 ms after the self identification process.

Thus the previous manager will stay bus manager because it will be the first node to send a request.

### 6. Determination of the CYCLE MASTER :

This is the bus manager that will decide which node will be the cycle master ( note : the cycle master has to be root in the cable specs of IEEE 1394 ). The bus manager ( or isochronous manager if there are no bus manager ) examines the bus_info_block of the root :

- If the root is cycle master capable :

If there are more than one node on the bus, the bus manager set the force_root bit of the root and clear the force_root bit of all other nodes. This is made by PHY configuration packets. If there

is only one node on the bus its force_root bit is cleared.

- If the root is not cycle master capable :

The bus manager selects one of the nodes that are cycle master capable , sets its force_root bit and initiates a bus reset. The node with its force_root bit set will become the root.

## IV. Data Packet Delivery :

The packet delivery services are the different services provided by the link layer. They include either asynchronous or isochronous data delivery. The main functions for these services are given bellow :

- When the data is sent, framing the packets in the good format. This includes the calculation of the CRC polynomials.

- Transmitting and receiving the packets.

- When the packets are received, checking received CRC polynomials and extracting the data from the frame.

### 1. Asynchronous packets :

All asynchronous packets share the same format. They consist of a header and sometimes of a data block. Each of these two blocks are followed by a CRC. The CRC are generated and checked using the algorithm defined by the IEEE 802 LAN standard ( Ethernet ).

The structure of an asynchronous data packet is illustrated bellow :

| Destination_ID[ 31 .. 16 ] | Tl[15..10] |  | Rt[9..8] | Tcode[7..4] | Pri[3..0] |
| --- | --- | --- | --- | --- | --- |
| Source_ID[ 31 .. 16 ] |  | Packet specific information ( Hi ) [ 15 .. 0 ] |  |  |  |
| Packet specific information ( Low ) [ 31 .. 0] |  |  |  |  |  |
| Packet specific data if any [ 31 .. 0] |  |  |  |  |  |
| Header CRC [ 31 .. 0 ] |  |  |  |  |  |
| Data block quadlet 1 [ 31 .. 0] |  |  |  |  |  |
| Data block quadlet 2 [ 31 .. 0] |  |  |  |  |  |
| … |  |  |  |  |  |
| Data block quadlet N [ 31 .. 0] |  |  |  |  |  |
| Data CRC [ 31 .. 0 ] |  |  |  |  |  |

The destination_ID and the source_ID are respectively the node_ID of the destination and source nodes. The value of the transaction code ( tcode ) specify the packet format ( asynchronous or isochronous ) and the type of transaction as following :

| Tcode | Name | comment |
| --- | --- | --- |
| 0 | Write request for data quadlet | Request subaction, quadlet payload |
| 1 | Writes request for data block | Request subaction, data block payload |
| 2 | Write response | Response subaction for both write requests, no payload |
| 4 | Read request for data quadlet | Request subaction, no payload |
| 5 | Read request for data block | Request subaction, quadlet payload |
| 6 | Read response for data quadlet | Response subaction to request for quadlet, quadlet payload |
| 7 | Read response for data block | Response subaction to request to block, block payload |
| 8 | Cycle start | Request to start isochronous cycle, quadlet payload |
| 9 | Lock request | Request subaction, block payload |
| A | Isochronous data block | Isochronous subaction, block payload |
| B | Lock response | Response subaction for lock request, block payload |
| Other | Reserved | -- |

There are different types of asynchronous packets with their own specific information used in specific circumstances, but most of the time it contains the destination offset.

Packet with no data payload :

- Read request for data quadlet : Request a single data quadlet from the destination_ID node.

- Read response for data quadlet : Sent in response to a read request for data quadlet.

- Write request for data quadlet : Request a data quadlet to be written to the destination_ID node

- Write response : Sent in response to a write request for data quadlet or data block.

- Read request for data block : Requests a data block from the destination_ID node.

- Cycle Start : Indicates the start of the isochronous cycle.

Packet with data payload :

The maximum size of the data payload is data rate dependent as detailed in the following board.

| Data rate | Maximum payload in bytes |
| --- | --- |
| S100 | 512 |
| S200 | 1024 |
| S400 | 2048 |

The different types of packet are :

- Write request for data block : Request a data block to be written to the destination_ID node.

- Lock request : Requests a locked access to the destination_ID node.

- Read response for data block : Sent in response to a read request for data block.

- Lock response : Sent in response to lock request.

### 2. Isochronous packets :

The format of the isochronous packet is always the same. The characteristic of these packets is that the node_ID of source and destination nodes are not given. It is the channel number that indicates to a node if the packet has to be received or not.

As for asynchronous packets, isochronous packets consist of a header with a header CRC followed by a data field and a data CRC.

| Data length[ 31 .. 16 ] | Tag[15..14] | Chan[13..8] | Tcode[7..4] | Sy[3..0] |
| --- | --- | --- | --- | --- |
| Header CRC [ 31 .. 0 ] |  |  |  |  |
| Data field [ 31 .. 0] |  |  |  |  |
| ... |  |  |  |  |
| Data CRC [ 31 .. 0 ] |  |  |  |  |

The size of the data field is not imposed by this standard, the only requirement is that the total amount of bandwidth used is not over the bandwidth available. This is done by the isochronous manager that allocates the appropriate amount of bandwidth to one node.

However assuming that only one node requires isochronous bandwidth, the maximum size of the isochronous packet at S100 speed is 1024 bytes.

### 3. Acknowledgement packets :

The acknowledgement packet is a single byte of data. The first four bits of this byte indicate the code of the acknowledgement ( won, lost, etc. ), the last four bits are the one's complement of the first four bits.

## V. Transactions :

The transaction are used by the transaction layer. The information communicated by these services are always asynchronous, and should be acknowledged in most cases. However if the transaction is broadcast, it is not acknowledged.

### 1. Transaction operation :

- Read transaction :

The application or bus manager require data to be received from an address in a different node.

- Write transaction :

The application or the bus manager require data to be transferred to an address in a different node.

- Lock transaction :

The application or the bus manager require data to be transferred to an address in a different node where this data must be used to perform an indivisible function and the result to be returned to the requesting node.

### 2. Types of transaction :

- Unified  transaction :

A unified transaction is a transaction that begins with an acknowledged request subaction and is not followed by a response subaction. Only write transactions can be unified.

- Split  transaction :

A split transaction is a transaction that begins with an acknowledged request subaction and is followed by an acknowledged response subaction. Write, Read and lock transactions can be spilt.

- Concatenated  transaction :

A concatenated transaction is a split transaction with no subaction gap between the request subaction and the response subaction. As for the spilt transaction, write, read and lock transactions can be concatenated.

- Broadcast  transaction :

A broadcast transaction is a request subaction that is not acknowledged. The physical_ID specified in the destination_ID is always 3Fh ( broadcast address ). All the nodes present on the bus should accept broadcast transactions and should not acknowledge it. Only write transaction should be broadcast.

## VI. Example of Data Transfer :

To illustrate the way the IEEE 1394 bus works we will take the example of a IEEE 1394 bus constituted of a PC, a camera and a DVD RAM, and details the steps of an asynchronous transfer, and also the different steps of an isochronous transfer.  We will suppose for both types of transfer that the bus has been successfully initialized ( reset, tree_ID, self_ID ).

figure 12. Bus configuration.

### 1. Example of Asynchronous Transfer :

Let's suppose that the DVD-RAM want to perform a write data block transaction to the PC, it must go through the following steps:

- Step 1 : Listen to the bus and wait for a subaction gap.

figure 13.

- Step 2 : Send a bus request ( TX_REQ ) to its parent node ( the PC ) if arb_enable flag is set ( Fairness arbitration ).

- Step 3 : The root of the bus ( the PC ) then arbitrates between the 2 nodes. In our example we will suppose that the Camera does not request the bus. Therefore the PC grants ( TX_GRANT ) the bus to the DVD_RAM and deny access ( with TX_DATA_PREFIX ) to the Camera.

- Step 4 : The DVD-RAM can then send its data packet to the PC. The transmission of this data packet ends with a data_end and the bus remains idle.

figure 14.

- Step 5 : The PC waits for a acknowledge gap and then sends after immediate arbitration ( always won ) an acknowledge packet for the data packet received to the DVD-RAM.

figure 15.

### 2. Example of Isochronous Transfer :

Let's suppose that the Camera send MPEG 2 data to the PC at a fixed rate of 6 Mbit/s. It is a typical case of isochronous transfer since the data must arrive on time and the bandwidth must be guaranteed. In order to achieve such a transfer, the Camera must go through the following steps :

- After the Bus initialization and prior to all its isochronous transfers the Camera must allocates bandwidth to the Isochronous Manager  of the bus ( the PC in our case ). This is done with a compare and swap operation on the bandwidth_available register of the isochronous manager. If there is enough bandwidth left ( we assume we are in this case ) then the bandwidth is reserved ( bandwidth_available register is decreased by the bandwidth reserved  by the camera ). The Camera can then allocate a channel for its isochronous data transmissions. To do so, the Camera does another compare and swap operation on the channels_available register of the isochronous manager to allocate one of the 64 channels available.  Once this is done, the Camera indicates the PC which channel it will use for its isochronous transfers with a write quadlet transaction. The isochronous transfers can then start at the next bus cycle.

- Step 1 : The Camera waits for the next cycle_start packet and then for an Isochronous gap.

figure 16.

- Step 2 : Send a bus request ( TX_REQ ) to its parent node ( the PC ) ( Normal arbitration ).

- Step 3 : The root of the bus ( the PC ) then arbitrates between the 2 nodes. In our example we will suppose that the DVD-RAM does not request the bus, since it doesn't generate isochronous traffic. Therefore the PC grants ( TX_GRANT ) the bus to the Camera and deny access ( with TX_DATA_PREFIX ) to the DVD-RAM.

- Step 4 : The Camera can then send its isochronous data packet to the PC. The transmission of this isochronous data packet ends with a data_end and the bus remains idle.

figure 17.

- Step 5 : The PC does not acknowledge this isochronous data packet. The Camera goes back to step 1 to send its next data packet. Each isochronous data packet from the camera represents 125s of MPEG2 data at 6 Mbit/s e.g. 94 bytes of data.

## VII. USB versus 1394 :

USB and IEEE 1394 are 2 bus standards that are shares several characteristics in common, but are somewhat different.  Indeed both of them are serial, hot plug and play buses. They all support both isochronous and asynchronous transfer modes, they uses almost the same kind of cable ( external aspect ). But their ends the comparison.

In fact this two buses target 2 different markets of the PC/Consumer Electronic industry : USB targets the low speed peripheral interconnect ( mice, etc. = parallel + I/O ports today )  market where as 1394 targets the high speed peripheral devices ( hard drives, scanner, etc. = SCSI today ). These difference in the target market leads to important differences in the internal specification of these 2 buses :

|  | USB | IEEE 1394 |
| --- | --- | --- |
| Maximum Transfer Rate | 12 Mbits / s | 400 Mbits / s |
| Maximum Number of Device | 127 | 63 per bus and 1023 buses |
| Inter-device max cable length | 5 m | 4.5 m |
| Power supply on Cable | 5V | 8 to 30 V |
| Number of pins per cable | 4 ( 1 twisted data pair, 2 power cables ) | 6 ( 2 twisted data pair; 2 power cables ) |
| Topology | Passive Tree with passive hubs, no leaf can manage the bus. | Active Tree with Active Controller at each node |
| Price Range | Current Serial and Parallel devices price range  ideal for generic PC connectivity | Current SCSI devices price range  ideal for Hard Drive, real time video devices. |

# B. Hardware Architecture of a generic IEEE 1394 Solution

## I. Global Project Overview :

The goal of this project is to develop an IEEE 1394 solution for generic deported application as shown if figure 18.

figure 18. IEEE 1394 Generic Application

The two 1394 controllers shown on the previous figure are based on the same architecture with only few differences in the modules due to specific client side bus interconnect ( PCI versus Local bus ).

## II. IEEE 1394 Controller functional block overview :

Thanks to IEEE 1394 Specifications and to OHCI version 1.0 drafts we have defined the following architectural drafts ( see figure 19 ) for our 1394 controller :

figure 19. IEEE 1394 Generic Controller Draft.

Note that the transaction layer is not implemented in hardware, but will be a software component of the controller to reduce hardware complexity. It will be part of the platform dependent driver of our controller.

## III. Block Level Detailed Architecture :

### 1. Phy Layer :

This Layer is responsible for Initialization, Arbitration at Physical level of the Bus, Raw Data transmission and Data Repeating. It knows all the rules for data transmission over an IEEE 1394 cable and all the constant necessary for data transfers. It contains internal registers that can be read and/or written by the link layer. In reality this layer contains two main parts : a state machine with internal registers and an analog part for data transmission in the differential mode required by IEEE 1394 specs.  An overview of the global architecture of this layer is shown in figure 20.

figure 20. Phy Layer Block details

### 2. Link Layer :

This Layer Provides Data Packet Delivery Services ( Framing packet, adding CRC, transmit & receive packets, checking receive packets CRC, extracting Data from Frame ). This provides also bus management services  such as cycle mastering, isochronous management, etc. The detailed block level architecture is shown in figure 21.

figure 21. Link Layer Block details

### 3. FIFO Controller :

This is just a set of FIFOs that syncs data from and to the IEEE 1394 bus with the Host Bus. It contains 3 FIFOS ( a Generic Receive FIFO,  an Asynchronous Transmit FIFO and an Isochronous Transmit FIFO ) and a controller. The detailed block level architecture is shown in figure 22.

figure 22. FIFO Controller Block details

### 4. Local Host Bus Adapter :

This layer adapts the signals from the FIFO controller with the local bus signals. The complexity of this module depend on the type of local bus to interconnect with. It can as simple as tri-state buffers for 68000 like local bus or as complicated as a complex module for PCI bus interconnect. At this point of design, no description of this block is available yet.

### 5. Software Component :

To reduce the hardware complexity of the controller, we have decided in our design that the transaction layer and part of the bus management layer will be developed in software. These components  will be part of the driver needed by the software embedded in the application to use the controller. Unfortunately no software specification are given in this report due to the lack of time.

## IV. Physical Layer Specification :

The Block Level architecture of the IEEE 1394 Phy layer described in the IEEE 1394 controller architecture is as follow ( see figure 23 ) :

figure 23.

### 1. Specs of the Cable Analog Interface :

This block is responsible of the analog conversion of the logical signals generated by Phy to differential voltage level required in IEEE 1394 protocol.  The Block Interface is described in figure 24.

figure 24.

TPA , TPB :  These are the 2 differential pairs as described in IEEE 1394 standard. The data are encoded on theses pair with the following voltage level :

| Logical Value : | Voltage : |
| --- | --- |
| 0 | > 168 mV |
| Z | Between -89 mV and 89 mV |
| 1 | < -168 mV |

Power : This is the power cable as defined in IEEE 1394 standard, the available voltage on this cable is between 8 and 40 V DC, and no more than 1.5 A can circulate on one link.

A_out :  This is a 3 states input pin  ( 0, 1, Z ) that represents the value to be output on TPA.

A_in : This is a 3 states output pin ( 0, 1, Z ) that represents the value read on TPA.

B_out :  This is a 3 states input pin  ( 0, 1, Z ) that represents the value to be output on TPB.

B_in : This is a 3 states output pin ( 0, 1, Z ) that represents the value read on TPB.

### 2. Specs of the Transmit Encoder Block :

This block knows how to generate the data and the strobe signals to be sent over the IEEE 1394 bus taking in input the data to be sent as well as the local clock. It also knows how to encode specific status code on TPA and TPB. The Block Interface is described in figure 25.

figure 25.

A_out :  This is a 3 states output pin  ( 0, 1, Z ) that represents the value to be output on TPA.

B_out :  This is a 3 states output pin  ( 0, 1, Z ) that represents the value to be output on TPB.

T_Data : These inputs represent the data to be transmitted on the bus if Data/Status pin is high. They are encoded as follow :

- 00  0, 01  Z, 10  1, 11  Invalid

Trans_clk :  The transmission clock input pin is the clock that will be used to send the data to the bus. This clock is active high on rising edge.

Data / Status :  This input pin indicates whether we are transmitting data ( Data/Status =  High ) or we are transmitting status ( Data/Status = Low ).

T_Status : These inputs represents the status to be transmitted  to the bus if Data/Status pin is low. Status are in format following bellow:

| T_Status ( Hex ) | Status | TPA, TPB values |
| --- | --- | --- |
| 0 | IDLE | Z, Z |
| 1 | TX_REQ | Z, 0 |
| 2 | TX_GRANT | Z, 0 |
| 3 | TX_PARENT_NOTIFY | 0, Z |
| 4 | TX_DATA_PREFIX | 0, 1 |
| 5 | TX_CHILD_NOTIFY | 1, Z |
| 6 | TX_IDENT_DONE | 1, Z |
| 7 | TX_DATA_END | 1, 0 |
| 8 | BUS_RESET | 1, 1 |
| 9 | Reserved | - |
| … | … | … |
| F | Reserved | - |

### 3. Specs of the Receive Decoder Block :

This block knows how to decode the data and the strobe on the bus. It also knows how to interpret the various states of the bus and knows which status code to output according to these states. The Block Interface is described in figure 26.

figure 26.

A_in : This is a 3 states input pin ( 0, 1, Z ) that represents the value read on TPA.

A_out :  This is a 3 states input pin  ( 0, 1, Z ) that represents the value to be output on TPA.

B_in : This is a 3 states input pin ( 0, 1, Z ) that represents the value read on TPB.

B_out :  This is a 3 states input pin  ( 0, 1, Z ) that represents the value to be output on TPB.

R_Data : These outputs represent the data received from the bus if Data/Status pin is high. They are encoded as follow :

- 00  0, 01  Z, 10  1, 11  Invalid

Recv_clk :  The reception clock output pin is the clock that was used to send the data on the bus. This clock is active high on rising edge.

Data / Status :  This input pin indicates whether we are receiving data ( Data/Status =  High ) or we are receiving status ( Data/Status = Low ).

R_Status : These outputs represents the status received from the bus if Data/Status pin is low. Status are in format following bellow:

| R_Status ( Hex ) | Status | TPA, TPB values |
| --- | --- | --- |
| 0 | IDLE | Z, Z |
| 1 | RX_PARENT_NOTIFY | Z, 0 |
| 2 | RX_REQ_CANCEL | Z, 0 |
| 3 | RX_IDENT_DONE | Z, 1 |
| 4 | RX_SELF_ID_GRANT | 0, Z |
| 5 | RX_REQUEST | 0, Z |
| 6 | RX_ROOT_CONTENT | 0, 0 |
| 7 | RX_GRANT | 0, 0 |
| 8 | RX_PARENT_HDSHK | 0, 1 |
| 9 | RX_DATA_END | 0, 1 |
| A | RX_CHILD_HDSHK | 1, Z |
| B | RX_DATA_PREFIX | 1, 0 |
| C | BUS_RESET | 1, 1 |
| D | Reserved | - |
| E | Reserved | - |
| F | Reserved | - |

### 4. Specs of the Phy State Machine :

This Block is the heart of the Phy layer. It is a state machine that knows how to arbitrate the bus on reset or for transmission purpose. It knows how to communicate with the Link layer Block via the Phy / Link bus.  The Block Interface is described in figure 27.

figure 27.

T_Data : These outputs represent the data to be transmitted to the bus if Data/Status pin is high. They are encoded as follow :

- 00  0, 01  Z, 10  1, 11  Invalid

T_clk :  The transmission clock output pin is the clock that will be used to send the data on the bus. This clock is active high on rising edge.

R_Data : These inputs represent the data received from the bus if Data/Status pin is high. They are encoded as follow :

- 00  0, 01  Z, 10  1, 11  Invalid

R_clk :  The reception clock output pin is the clock that was used to send the data on the bus. This clock is active high on rising edge.

Data / Status :  This output pin indicates whether we are transmitting / receiving data ( Data/Status =  High ) or we are transmitting / receiving status ( Data/Status = Low ).

T_Status : These outputs represents the status to be transmitted  to the bus if Data/Status pin is low.

R_Status : These inputs represents the status received from the bus if Data/Status pin is low.

Receive : This output indicates the Link layer that data are being received from the bus.

Transmit_Granted :  This output indicates the Link layer that a previous transmit request has been granted.

Hold : This input indicates the Phy layer that the Link Layer is preparing the data to be sent after the bus was granted and that the Phy layer should try to hold the bus as long as possible.

Link_req : This input indicates the Phy layer that the bus layer want to send data on the bus. A bus request must be sent in the command register of the Phy layer concurrently to the activation of this line.

Link_DS : This input indicates that the data on Link_data  and the address on Add are strobe and can be sampled by the Phy Layer.

R/W :  This input indicates whether the Link want to execute a read ( R/W = High ) or a Write ( R/W = Low ) operation.

Add : This 8 bits input specifies the address where the next read or write operation should take place.

Link_Data :  This 8 bits input represents the data to be written in the Phy Layer by the Link layer.

Phy_DS : This output indicates that the data on Phy_Data are strobe and can be  sampled by the Link layer.

Phy_Data : This 8 bits output represents the data to be written in the Link Layer by the Phy Layer.

Clk : This is the Phy/Link clock, it is a 49.152 Mhz Clock as defined in the IEEE 1394 standard.

Link_On : This output indicates the Link Layer that the Link is on the IEEE 1394 cable.

Power_Down : This input indicates the Phy Layer that the device is off.

/Reset :  This input is the general reset input. It will reset the Phy layer and may resets the bus.

The Phy State machine contains internal registers that can be accessed by the Link layer. Here follows the description of these registers :

| Access | Address( Hexa ) | Register name and Topology |
| --- | --- | --- |
| R | 00 | T/R_Status = ¦¦ T_Status[7..4] ¦¦ R_Status[3..0] ¦¦ |
| R | 01 | Node_Conf = ¦¦ Child(7) ¦¦ Root(6) ¦¦ Phy_Id[5..0] ¦¦ |
| R | 02 | Cable_Port_Status = ¦¦ Cable_On(7) ¦¦ Cable_Power_Status(6) ¦¦ N_port[5..2] ¦¦ Speed_Max[1..0] ¦¦ |
| R/W | 03 | Gap_Count_Register = ¦¦ Gap_Counter[7..0] ¦¦ |
| R/W | 04 | Interrupt_Reg = ¦¦ Cable_Power_Low(7) ¦¦ Loop_Timeout(6) ¦¦ Reserved[5..0] ¦¦ |
| R/W | 05 | Reset_Management_Reg = ¦¦ Initiate_Bus_Reset(7) ¦¦ Reset_Initiated(6) ¦¦ Root_Hold_Off(5) ¦¦ Reserved[4..0] ¦¦ |
| R/W | 06 | Command Register = ¦¦ Com_Reg[7..0] ¦¦ |
| R/W | 07 | Transmit_Data_Reg = ¦¦ TX_DATA[7..0] ¦¦ |
| R | 08 | Phy Version = ¦¦ version[7..0] ¦¦ |

The Phy State Machine states are not described in this paper as they are too close from implementation level. Details on this state machine may be added latter in this paper, otherwise please refer to the phy state machine C++ code in the software version of the controller.

## V. Link Layer Specification :

The Block Level architecture of the IEEE 1394 Link layer described in the IEEE 1394 controller architecture is as follow ( see figure 28 ) :

figure 28.

### 1. Specs of the Phy Interface :

This block interfaces the internal signals from the transmitter, the state machine and the receiver with the Phy Layer. The Block Interface is described in figure 29.

figure 29.

Link_DS : This output indicates that the data on Link_data  and the address on Link_Add are strobe and can be sampled by the Phy Layer.

Link_R/W :  This output indicates whether the Link want to execute a read ( R/W = High ) or a Write ( R/W = Low ) operation.

Link_Add : This 8 bits output specifies the address where the next read or write operation should take place.

Link_Data :  This 8 bits output represents the data to be written in the Phy Layer by the Link layer.

Phy_DS : This input indicates that the data on Phy_Data are strobe and can be  sampled by the Link layer.

Phy_Data : This 8 bits input represents the data to be written in the Link Layer by the Phy Layer.

Receive : This input indicates the Link layer that data are being received from the bus.

Transmit_Granted :  This input indicates the Link layer that a previous transmit request has been granted.

Hold : This output indicates the Phy layer that the Link Layer is preparing the data to be sent after the bus was granted and that the Phy layer should try to hold the bus as long as possible.

Link_request : This output indicates the Phy layer that the bus layer want to send data on the bus. A bus request must be sent in the command register of the Phy layer concurrently to the activation of this line.

Clk : This is the Phy/Link clock, it is a 49.152 Mhz Clock as defined in the IEEE 1394 standard.

Link_On : This input indicates the Link Layer that the Link is on the IEEE 1394 cable.

Power_Down : This output indicates the Phy Layer that the device is off.

/Reset :  This output is the general reset output It will reset the Phy layer and may resets the bus.

Transmit_DS : This input pin indicates that the data on Transmit_Data are strobe and must be transmit to the Phy Layer.

Transmit_Data : This 8 bits input pins are the data packet to be sent to the Phy Layer.

Control_R/W : This input indicates if the Link_State_Machine want to perform a read or a write operation on the Phy Layer.

Control_Add  : This 8 bits input contains the Address where the next Read or Write from the Link State Machine must take place.

Control_Dout_DS : This input line indicates that the data from the Link State Machine to be output to Phy are strobe.

Control_Dout : This 8 bits input contains the data from the Link State Machine to be output to Phy.

Control_Din_DS : This output line indicates that the Phy Layer is sending control data and that they are strobe on Control_Din.

Control_Din : This 8 bits output contains the control data sent by the Phy Layer to the Link State Machine.

Receive_DS : This output pin indicates that Phy is sending data packet to the Link Layer ( receive is high )

Receive_Data : This 8 bits output pins are the data packet received from phy.

### 2. Specs of the Link  Transmitter :

This block is responsible for data transmission rate management, CRC encoding and packet framing.

It basically reclocks the data packet and send them at the correct speed to the Phy Layer. It also adds the CRC code at the end of the Header and at the end of the Data section. The Block interface is described in figure 30.

figure 30.

Transmit_DS : This output pin indicates that the data on Transmit_Data are strobe and must be transmit to the Phy Layer.

Transmit_Data : This 8 bits input pins are the data packet to be sent to the Phy Layer.

Transmit_packet : This input indicates that the State Machine is transmitting a packet. Simultaneously to this Speed_code must contain the transmission speed and Frame position must be set to  header_pos.

Speed_code : This 2 bits input contains the speed code of the speed at which the packet will be sent . The different code are the following :

| Speed Code ( binary ) | Effective Speed ( Mbit / s ) |
| --- | --- |
| 00 | 98.304 |
| 01 | 196.608 |
| 10 | 393.216 |
| 11 | 786.432* |

- *not implemented yet

Frame_pos : This 2 bits input indicates which part of the packet is being transmitted at the moment. The encoding of this bits are the following :

| Frame_pos ( binary ) | Meaning |
| --- | --- |
| 00 | Header |
| 01 | Data |
| 10 | Reserved |
| 11 | EOF |

Data : The 32 bits data to be added to the frame.

Next_data_quad : This output indicates the Link State Machine that it must send the next 32 bits of data at the next rising edge of the clock.

CRC_data_out : This output indicates to the CRC module that the next 32 bits of CRC_data are strobe. It must be maintained high for one clock cycle.

CRC_generate : This output indicates the CRC module that it must generate a 32 bits CRC pattern.

CRC_data : This 32 bits output contains the data sent to the CRC module.

CRC_pattern : This 32 bits input contains the CRC pattern generated by the CRC module when CRC_pattern_ready pin is high.

CRC_pattern_ready : This input indicates the transmitter that the CRC pattern has been generated and is strobe on CRC_pattern pin.

### 3. Specs of the CRC Module :

This module knows how to generate IEEE 1394's 32 bit CRC for the Header and for the Data section. It generates this 32 CRC incrementally until CRC_generate is activated. Then the CRC_pattern is output and CRC_ pattern ready is set high until the CRC_data_in is asserted high. The Block interface is described in figure 31.

figure 31.

CRC_data_in : This input indicates to the CRC module that the next 32 bits of CRC_data are strobe. It must be maintained high for one clock cycle.

CRC_generate : This input indicates the CRC module that it must generate a 32 bits CRC pattern.

CRC_data : This 32 bits input contains the data sent to the CRC module.

CRC_pattern : This 32 bits output contains the CRC pattern generated by the CRC module when CRC_pattern_ready pin is high.

CRC_pattern_ready : This input indicates the transmitter that the CRC pattern has been generated and is strobe on CRC_pattern pin.

### 4. Specs of the Link Receiver :

This module is responsible for data reception rate management, CRC checking and data filtering ( by checking the Destination ID ). The Block interface is described in figure 32.

figure 32.

Receive_DS : This input pin indicates that Phy is sending data packet to the Link Layer.

Receive_Data : This 8 bits input pins contains the data packet received from phy.

Receive : When asserted high this output pin indicates the Link State Machine that a data packet is being receive.

Data_quad_strobe : When asserted high this output pin indicates the Link State Machine that it must read the a data quadlet at the next rising edge of the clock.

Data_quad : This 32 bits output pins contains the data quad being received from Phy when Data_quad_strobe is asserted high.

Physical_ID : This 16 bits input pin contains the Unique Physical ID of the device : ( Bus_ID[15..6], Node_ID[5..0] ).

Check_CRC : When asserted high this input pin indicates the receiver that it must start checking the CRC of the block previously received before the next rising edge of the clock.

CRC_check_done :  This output pin indicates the Link State Machine that the CRC check has been done and that the result is available on the CRC_correct pin.

CRC_correct : This output pin indicates the result of the last CRC check. When asserted high it indicates that the last CRC check was correct.

CRC_data_out : This output indicates to the CRC module that the next 32 bits of CRC_data are strobe. It must be maintained high for one clock cycle.

CRC_generate : This output indicates the CRC module that it must generate a 32 bits CRC pattern.

CRC_data : This 32 bits output contains the data sent to the CRC module.

CRC_pattern : This 32 bits input contains the CRC pattern generated by the CRC module when CRC_pattern_ready pin is high.

CRC_pattern_ready : This input indicates the transmitter that the CRC pattern has been generated and is strobe on CRC_pattern pin.

### 5. Specs of the Isochronous Manager :

This module holds the cycle timer register as defined in IEEE 1394 specifications. This register can be updated by cycle_start packet and incremented by the local 24.59 Mhz clock or it can be entirely managed by the local node if node is isochronous cycle master. The Block interface is described in figure 33.

figure 33.

Cycle_master : When asserted high this input pin indicates that the node is cycle master and must generate the cycle start packet of the IEEE 1394 bus.

Update : When asserted high it indicates that the Cycle Timer register of the isochronous manager must be updated with the Cycle Start packet available on the Cycle_start_quad 32 bits pin.

Cycle_start_quad : this 32 bits pin contains the cycle start quadlet when the Update pin is asserted high.

Cycle_timer_quad : This 32 bits output pin contains the Cycle Timer register value permanently.

Send : This output pin indicates the Link State Machine that a new cycle start must be sent on the bus. It is used only if the node is Isochronous cycle master.

### 6. Specs of the Isochronous Monitor :

This module monitors the isochronous cycles on the bus and plans the isochronous transmissions according to the Isochronous Transmission FIFO state. The Block interface is described in figure 34.

figure 34.

Cycle_timer_quad : This 32 bits input pin contains the value of the cycle timer register from the isochronous manager.

Channel_reserved : This 64 bits input pins indicates the channels officially reserved by this node.

Isoch_bus_req : This output pin indicates the Link Layer State Machine that it should request the bus to send the isochronous packet pending on channel Isoch_channel.

Isoch_bus_req : This input pin indicates if the pending isochronous bus request has been granted. If so Isoch_bus_req is reset, and will be set again if another isochronous packet must be sent in this cycle.

Isoch_channel :  This 6 bits output pin indicates the ID of the channel requested.

### 7. Specs of the Link Layer State Machine :

This module is the heart of the Link Layer, It holds all the registers defined in IEEE 1394 standard, it knows how to handle data packets, how to answer transmission requests from the FIFO Controller. The different states of this machine will not be defined in this paper. For further description please refer to the VHDL code on the link state machine. The block interface of this state machine is described in figure 35.

figure 35.

Control_R/W : This output indicates if the Link_State_Machine want to perform a read or a write operation on the Phy Layer.

Control_Add  : This 8 bits output contains the Address where the next Read or Write from the Link State Machine must take place.

Control_Dout_DS : This output line indicates that the data from the Link State Machine to be output to Phy are strobe.

Control_Dout : This 8 bits output contains the data from the Link State Machine to be output to Phy.

Control_Din_DS : This input line indicates that the Phy Layer is sending control data and that they are strobe on Control_Din.

Control_Din : This 8 bits input contains the control data sent by the Phy Layer to the Link State Machine.

Transmit_Granted :  This input indicates the Link layer that a previous transmit request has been granted.

Hold : This output indicates the Phy layer that the Link Layer is preparing the data to be sent after the bus was granted and that the Phy layer should try to hold the bus as long as possible.

Link_request : This output indicates the Phy layer that the bus layer want to send data on the bus. A bus request must be sent in the command register of the Phy layer concurrently to the activation of this line.

Transmit_packet : This output indicates that the State Machine is transmitting a packet. Simultaneously to this Speed_code must contain the transmission speed and Frame pos must be set tot header_pos.

Speed_code : This 2 bits output contains the speed code of the speed at which the packet will be sent by the transmitter . The different code are the following :

| Speed Code ( binary ) | Effective Speed ( Mbit / s ) |
| --- | --- |
| 00 | 98.304 |
| 01 | 196.608 |
| 10 | 393.216 |
| 11 | 786.432* |

- *not implemented yet

Frame_pos : This 2 bits input indicates which part of the packet is being transmitted at the moment by the State Machine to the transmitter. The encoding of this bits are the following :

| Frame_pos ( binary ) | Meaning |
| --- | --- |
| 00 | Header |
| 01 | Data |
| 10 | Reserved |
| 11 | EOF |

Transmit_data : The 32 bits data to be added to the frame by the transmitter.

Transmit_next_data_quad : This input indicates the Link State Machine that it must send the next 32 bits of data to the transmitter at the next rising edge of the clock.

Receive : When asserted high this input pin indicates the Link State Machine that a data packet is being receive.

Data_quad_strobe : When asserted high this input pin indicates the Link State Machine that it must read the a data quadlet at the next rising edge of the clock.

Data_quad : This 32 bits input pins contains the data quad being received from Phy when Data_quad_strobe is asserted high.

Physical_ID : This 16 bits output pin contains the Unique Physical ID of the device : ( Bus_ID[15..6], Node_ID[5..0] ).

Check_CRC : When asserted high this output pin indicates the receiver that it must start checking the CRC of the block previously received before the next rising edge of the clock.

CRC_check_done :  This input pin indicates the Link State Machine that the Receive CRC check has been done and that the result is available on the CRC_correct pin.

CRC_correct : This input pin indicates the result of the last Receive CRC check. When asserted high it indicates that the last Receive CRC check was correct.

FIFO_DS : This input indicates that the data on FIFO_data  and the address on FIFO_Add are strobe and can be sampled by the Link Layer.

FIFO_R/W :  This input indicates whether the FIFO Controller want to execute a read ( R/W = High ) or a Write ( R/W = Low ) operation.

FIFO_Add : This 8 bits input specifies the address where the next read or write operation should take place in Link Layer.

FIFO_Data :  This 32 bits input represents the data to be written in the Link Layer by the FIFO Controller.

Link_DS : This output indicates that the data on Link_data  and the address on Link_Add are strobe and can be sampled by the FIFO Controller.

Link_R/W :  This output indicates whether the Link want to execute a read ( R/W = High ) or a Write ( R/W = Low ) operation .

Link_Add : This 3 bits output specifies the address where the next read or write operation should take place in the FIFO Controller.

Link_Data :  This 32 bits output represents the data to be written in the FIFO Controller by the Link layer.

This State Machine must also hold in addition to its operation registers the registers defined in the IEEE 1394 standards.  These registers can be accessed through the IEEE 1394 bus at the base address FFFF F000 0000 h. The list of these registers is as follow :

Register Name : NODE_IDS			Offset : 008 h

Usage : This register holds the current bus ID and current physical ID of this node.

| Bus_id ( 10 bits ) | Phy_id ( 6 bits ) | Zeros ( 16 bits ) |
| --- | --- | --- |

Register Name : RESET_START			Offset : 00C h

Usage : A write to this register performs an immediate reset.

Register Name : SPLIT_TIMEOUT		Offset : 018 h

Usage : Specifies the maximum time a node waits for a response subaction after a request subaction. The transaction is aborted after this time.

| Zeros ( 29 bits ) |  | Sec ( 3 bits ) |
| --- | --- | --- |
| Fraction of second in 125us ticks ( 13 bits ) | Zeros ( 19 bits ) |  |

Register Name : CYCLE_TIME			Offset : 200 h

Usage : This register is nothing but the Cycle time register hold by the Isochronous manager of the node.

| Sec_count ( 7 bits ) | Cycle_count ( 13 bits ) in 125us ticks | Cycle_offset ( 12 bits ) in 25.576 Mhz clock ticks |
| --- | --- | --- |

Register Name : BUS_TIME			Offset : 204 h

Usage :  Holds a measure of the current time in second and does not reinitialize after a bus reset.

| Second_count_hi ( 25 bits ) | Second_count_low |
| --- | --- |

Register Name : BUSY_TIMEOUT		Offset : 210 h

Usage : Is used to control the number of retries. The maximum number of retries is given by retry_limit.

| Reserved ( 4bits ) | S_lim | Cycle_limit ( 13 bits ) | Reserved ( 8 bits ) | Retry_limit ( 4 bits ) |
| --- | --- | --- | --- | --- |

Register Name : BUS_MANAGER_ID		Offset : 21C h

Usage :  Gives the physical ID of the current bus manager. The initial value of this field is 3F h .

| Reserved ( 26 bits ) | Bus_mngr_id |
| --- | --- |

Register Name : BANDWIDTH_AVAILABLE	Offset : 220 h

Usage :  Holds the bandwidth remaining that can be allocated for isochronous transfer, it is expressed in unit equal to the time need to transfer a data quadlet at S1600 speed. The initial value of Bw_remaining is 4915 .

| Reserved ( 19 bits ) | Bw_remaining ( 13 bits ) |
| --- | --- |

Register Name : CHANNELS_AVAILABLE		Offset : 224 h

Usage : Indicates the channels that are currently used for isochronous transfer. A zero at a given position indicates that the corresponding channel is allocated.

Channels_available_hi ( 32 bits )

Channels_available_low ( 32 bits )

Register Name : TOPOLOGY_MAP			Offset : 800 h

Usage : This register is the registration of the self_ID packets observed. This map may be used to optimize the gap_count value. The length field specifies the number of following quadlets. The CRC field covers all the following quadlets. The node_count field gives the number of nodes present on the local bus. The  self_id_count field gives the number of self_ID packets following.

| Length ( 16 bits ) | CRC ( 16 bits ) |
| --- | --- |

Generation_number

| Node_count ( 16 bits ) | Self_id_count ( 16 bits ) |
| --- | --- |

Self_ID packet [ 0.. Self_id_count - 1] ( 32 bits )

# C. Validation Method : My 1394 Simulator

In order to validate our hardware implementation we will describe a validation method that sounds realistic enough for us and that will allow us to check the validity of our implementation. No that these validation method will only check the hardware at conception level, but any case will the real hardware implementation be tested. No validation procedure will be described here to validate the hardware after the conception phase.

Our validation method are based on two main parts:

- A 1394 Simulator that will be able to simulate different bus configuration and that will generate Bus Event Log files

- A 1394 Checker that will be able to checks Event log File to find out if all the events happening on the Bus are compliant with the IEEE 1394 Specifications.

To validate our hardware at the conception stage, we will build a VHDL simulation environment that will also generate a bus event log file in a format similar to the one generated by the 1394  Simulator. The 1394 Checker will then check this file to find out if everything is working normally on the bus constituted of VHDL models of our controller according to IEEE 1394 Specifications.

The two main components of  our validation techniques will be described farther in the next two chapters.

## I. 1394 Software Simulator :

The software simulator has two main objectives. First it provides a software model for the IEEE 1394 Controller that will be useful in the development of the hardware version. Second its Event Log files will be used to debug the 1394 Checker.

The simulator is constituted of two main components, a software version of the IEEE 1394 controller and a simulator that simulates the activities of a bus constituted of these controllers.

### 1. Software controller :

The software controller is a C++ model of the 1394 Hardware controller described in part B of this report. We broke the controller into five main classes :

C1394_Phy :

This class is the equivalent of the Physical layer in the hardware model. It can simulate the phy layer activity and report the main events of this layer in the Event Log file. It knows all the initialization and arbitration rules of the Bus, it holds all the timing constants, etc. The main public method of this class  is the simulate_10ns which simulates 10ns of activity of the PHY layer. The prototype of this method is as follows :

void simulate_10ns( unsigned long date, FILE *event_log, TP *port, PL_Interface * pl_int );

date : the simulation current date.

event_log : the event log file descriptor.

port :  is a pointer to a set of data transmission pairs that represents the sates of the port a this simulation date.

pl_int : is a pointer to a PL_Interface structure that represents the signals exchanged between the Phy and the Link layer.

For more details regarding this class, its header file is available in annexes.

C1394_Link :

This class is the equivalent of the Link Layer plus the FIFOs in the hardware model. It can simulate the link layer activity and report the main events of this layer in the Event Log File. It knows how frame and unframe data packets, how to compute and check CRCs. It is also able to generate cycle_start packet and to monitor isochronous cycle and to schedule isochronous transmission. It uses a C1394_Phy object to perform its physical transmission tasks. The header file of this class is also available in annexes.

C1394_Trans :

This class is responsible for the simulation of all the transaction management. It can simulate the transaction layer activity and report the main events of this layer in the Event Log File. It knows how to handle all the different kind of transactions normalized in the IEEE 1394 standard. It uses a C1394_Link object to access the IEEE 1394 bus. The header file of this class is available in annexes.

C1394_Controller :

This class represents a whole IEEE 1394  Controller. It uses both a C1394_Trans object for asynchronous transactions and the C1394_Link object used by this C1394_Trans object for isochronous transmissions. It knows how to initiate and to generate isochronous data transfers, it emulates a Host system memory for the different asynchronous transactions. It simulates thanks to all the former class the activity of the whole controller and generates Event Logs if needed. The header file of this class is available in annexes.

C1394_Network :

This class maintains a collection of IEEE 1394 Network of controller objects that are plugged according to the information contained in a configuration file, where are also described the kind of traffic generated by each node of the bus, the simulation length and the name of the event log file to be generated. The C1394_Network simulates a network operating at 100Mbit/s speed. The header file of this class is available in annexes.

### 2. 1394 Simulator :

The Simulator is simply an interface to access a C1394_Network object. It takes in input the name of the configuration file and then creates a C1394_Network object for the simulation. A graphical version  ( Windows ) of the simulator will hopefully be demonstrated the day this report will be defended.

## II. 1394 Checker :

The 1394 checker is the main piece of our 1394 Validation Process, since it is the program that will verify  the event log files and check if the event happening on the bus does not violate the IEEE 1394 specifications. The 1394 checker has not been implemented due to lack of time but here are the main point it should handle :

- It must know how to read the event log files that will be generated either by the 1394 Simulator or by the IEEE 1394 VHDL Simulation environment.

- It must have a base of rules that handle the different specifications of the bus in a simple language. This means that this base of rule will be easily maintainable and is not hard coded with the checker.

- It must check the activity of all the node . This means, it should check that all the gaps on the bus are conform to the specifications, that the CRC encoding ( of the link layer ) is correct, that the initialization steps are correct, etc.

This is in our opinion the functionality required  to check the design of our controller, since it is not possible to do a classic functional  check ( where you input data to the controller and check if the output data from the controller are what you expect ) as the controller is not only an operator ( like a 64bits adder ) but a complex state machine .

# Conclusion

- The main objectives of this report were to lead the reader to a complete understanding of the IEEE 1394 bus specifications, to presents architectural specifications for a hardware implementation of a generic IEEE 1394 controller and to introduce validation methods needed to check this kind of hardware development. We hope we have achieved these objectives.  Although we didn't go farther in the implementation  of this controller ( by lack of time ), we hope that the developer who will read this report will have, thanks to our hardware implementation  specifications, a good starting point for its design and validation phase.

- In our view the required steps for a designer who wants to build its own IEEE 1394 Controller based on our specifications, will be first to have a complete understanding of the 1394 Bus. Second to builds its own hardware specifications based on the ones built in this report ( to correct eventual conception flaws ). Third to build its software specifications for the software part of the controller. Fourth to start coding the hardware controller in a hardware description language such as VHDL. Fifth to validate its hardware controller design thanks to the validation methods described in this report or thank to any others applicable method. Sixth to synthesize the hardware code for its target ( FPGA or others ). Seventh to build its final testing environment. Eighth to writes the software pieces of the controller such as the transaction layer. Ninth   to write the software drivers needed to run its controller according to its testing environment. And eventually tenth, to test and validate its entire controller ( hardware + software ). View the complexity of the work remaining and the different skills needed, it seems almost compulsory to us that both, a hardware designer and a software driver programmer  work closely together in order to realize such a project.

- Finally, this "mémoire" on the IEEE 1394 Bus allowed us to discover a very well designed bus specification. It leads us through the understanding of a very large and complex bus standard and through the formal design of a hardware architecture. Unfortunately the lack of time put a end to this adventure for us, but we hope this end will be a good starting point for other 1394 addicts.

# Annexes

## I. Tree ID Process :

At the beginning of the tree identification process, the only thing a node "knows" is if it is whether a leaf or a branch. All cables are in state idle. The different steps of the tree_ID process are described bellow :

- Step 1 : All leaf nodes send a parent_notify ( Z0 )

- Step 2 : A node that has received parent_notify on at least all but one port sends

- Parent_notify to the remaining port.

- Child_notify ( 1Z ) to the ports that have sent the parent_notify.

- Step 3 : When a node receives a child_notify in response to its parent notify, it

knows that the node that has sent the child_notify is truly its parent. The node then withdraws its parent_notify and its child notify if any.

If two next nodes sent a parent_notify to each other, the line sate is 00 which correspond to a root_contention. These two nodes then enter in a new process to determine the root. This process begins  by withdrawing their parent_notify. The line is then in state IDLE.

- Root Contention Process : The nodes implicated in a root contention start a timer,

with time chosen randomly between two values ( ROOT_CONTENTION_LOW and ROOT_CONTENTION_FAST ). When the time expires, each node make a check of the state of the line :

If the line is still idle it sends a parent_notify and a new root_contention may occur. If the line is in state RX_PARENT_NOTIFY it becomes the parent node and sends a child_notify to the other port.

When the node implicated in the contention root receives a child_notify, it withdraws its parent_notify and child_notify as specified in the step 3.

- Step 4 : The node that sends child_notify to all its ports knows it is the root. It

then withdraws its child_notify and is marked as the root. After this, the root begins the self identification

process.

## II. 1394_Phy.h :

/*--------------------------------------------------------------------

|

|      1394 Phy layer header file

|      by ralao ( ENST 1998 )

|

+--------------------------------------------------------------------*/

#ifndef _1394_PHY_H_

#define _1394_PHY_H_

/*--------------------------------------------------------------------

|	Include definitions bellow this line

+--------------------------------------------------------------------*/

#include <stdio.h>

/*--------------------------------------------------------------------

|	Constant definitions bellow this line

+--------------------------------------------------------------------*/

/*** Number of PHY registers and registers names ***/

#define N_PHY_REG           9

#define TRS_REG             0       // Transmit and Receive Status Reg

#define NC_REG              1       // Node Configuration Reg

#define CPS_REG             2       // Cable Port Status Reg

#define GC_REG              3       // Gap Count Reg

#define INT_REG             4       // Interrupt Reg

#define RM_REG              5       // Reset Management Reg

#define CR_REG              6       // Command Register

#define TD_REG              7       // Transmit Data Reg

#define PV_REG              8       // Phy Version Reg

/*** Gaps in nanoseconds ***/

#define ACK_GAP             40      // Acknowledge gap 0.04us

#define ISOC_GAP            40      // Isochronous gap 0.04us

#define BASE_RATE           0.1     // Base Frequency in Ghz

#define DEF_GAP_COUNT       63      // Default gap count value

#define DEF_SUBA_GAP        ( 27 + GAP_COUNT_DEF * 16 ) / BASE_RATE

#define DEF_ARB_RESET_GAP   ( 51 + GAP_COUNT_DEF * 32 ) / BASE_RATE

#define ARB_DELAY           GAP_COUNT_DEF * 4 / BASE_RATE

/*** other timing constants in ns ***/

#define ACK_RESPONSE_TIME   50      // Time between Data end and ACK

#define CONFIG_TIMEOUT      166666L // loop detect time

#define ROOT_CONTEND_FAST   240     // wait time during root contention fast

#define ROOT_CONTEND_SLOW   570     // wait time during root contention slow

#define FORCE_ROOT_TIMEOUT  83333L  // wait time before acknowledging parent_notify

#define NOMINAL_CYCLE_TIME  124988L // average length of a cycle

#define DATA_PREFIX_TIME    40      //

#define MAX_BUS_OCCUPANCY   100000L // maximum time a node may transmit

#define RESET_TIME          166666L // reset hold time

#define RESET_WAIT          166800L // reset wait time

/*--------------------------------------------------------------------

|  Type definitions bellow this line

+--------------------------------------------------------------------*/

/*** TP : Transmission Pairs ***/

typedef struct {

char A;  // TPA can be '0', '1', 'Z'

char B;  // TPB can be '0', '1', 'Z'

} TP;   // Transmission Pairs

/*** TX_State ***/

typedef enum {

TX_IDLE,            // indicates a gap

TX_REQUEST,         // sends a Bus request

TX_GRANT,           // notifies a bus grant

TX_PARENT_NOTIFY,   // Tree ID specific signal

TX_DATA_PREFIX,     // data packet prefix

TX_CHILD_NOTIFY,    // Tree ID specific signal

TX_IDENT_DONE,      // indicates Self ID completion

TX_DATA_END,        // indicates data packet end

TX_BUS_RESET        // forces bus reset

} TX_State; // Transmitted State

/*** RX_State ***/

typedef enum {

RX_IDLE,            // attached node is inactive

RX_PARENT_NOTIFY,   // Tree ID specific signal

RX_REQ_CANCEL,      // attached node has canceled its request

RX_IDENT_DONE,      // attached has finished self ID

RX_SELF_ID_GRANT,   // bus is granted for Self ID

RX_REQUEST,         // child node is requesting the bus

RX_ROOT_CONTENTION, // ROOT_CONTENTION

RX_GRANT,           // bus has been granted

RX_PARENT_HS,       // attached node acknowledges parent notify

RX_DATA_END,        // attached node has finished sending data

RX_CHILD_HS,        // attached node acknowledges child notify

RX_DATA_PREFIX,     // attached node is about to send a packet

RX_BUS_RESET        // bus must be reset

} RX_State; // Received State

/*** Phy_State ***/

typedef enum {

PS_IDLE,        // Idle state

PS_RESET,       // Bus reset state

PS_TREE_ID,     // Tree ID State ( this state contains sub-states )

PS_SELF_ID,     // Self ID State ( this state contains sub-states )

PS_ARB,         // Arbitration state

PS_TX,          // Transmission state

PS_RX,          // Reception state

PS_FWD          // Froward Data from the receiving node to the other nodes

} Phy_State;    // Phy State machine states

/*** PL_Interface ***/

typedef struct {

bool    p_receive;  // indicates to link that phy is receiving data

bool    p_tx_grant; // indicates to link that bus is granted

bool    p_link_on;  // indicates to link that cable link is on

bool    p_ds;       // indicates that phy data are strobe

char    p_data;     // data from phy

bool    l_bus_req;  // indicates to phy that link is requesting the bus

bool    l_hold;     // indicates to phy that it must hold the bus

bool    l_pwr_down; // indicates to phy that the power is down

bool    l_reset;    // indicates  that phy must reset ( active low )

bool    l_ds;       // indicates that link data are strobe

bool    l_rnw;      // true = reading, false = writing

char    l_add;      // address where link want to read or write

char    l_data;     // data from link in case of a write

} PL_Interface; // Phy Link Interface signals

/*--------------------------------------------------------------------

|  Class definitions bellow this line

+--------------------------------------------------------------------*/

/*** 1394 Phy layer class definition ***/

class C1394_Phy

{

protected :   // private attributes bellow this line

Phy_State   m_state;        // current state of he phy state machine

TP          *m_port_tx;     // array of ports ( transmitted values )

TP          *m_port_rx;     // array of ports ( received values )

char        m_reg[N_PHY_REG];   // internal registers of PHY (read/write)able from LINK

public :    // public attributes bellow this line

bool        m_arb_enable;   // fairness arbitration erb_enable indicator

bool        m_force_root;   // indicates if the node must become root at the next reset

bool        m_init_reset;   // indicates if the node has initiated the reset process

bool        m_link_active;  // indicates if the link layer is active

int         m_n_port;       // number of port of this phy layer

int         m_p_port;       // parent port

int         m_r_port;       // receive port : port that is currently receiving data

int         m_gap_count;    // gap count value

int         m_phy_id;       // phy_id of this node

int         m_logical_id;   // logical id of this node, used for simulation purpose

int         m_step;         // steps done in the current state

unsigned long m_idle_time;  // bus idle time;

// public constructor

C1394_Phy( int logical_id, int n_port = 1 );

// public virtual destructor

virtual ~C1394_Phy();

// resets the phy layer

void reset( void );

// transforms a tx state value to a transmission pair value

TP  txs2tps( TX_State txs);

// transforms a transmission pair value to a tx state value

RX_State  tps2rxs( TP tps);

// simulates 10ns of the phy layer activity

// reports the important event happening in phy to the event_log

void simulate_10ns( unsigned long date, FILE *event_log, TP *port, PL_Interface * pl_int );

};

#endif // _1394_PHY_H_

## III. 1394_Link.h :

/*--------------------------------------------------------------------

|

|      1394 Link layer header file

|      by ralao ( ENST 1998 )

|

+--------------------------------------------------------------------*/

#ifndef _1394_LINK_H_

#define _1394_LINK_H_

/*--------------------------------------------------------------------

|	Include definitions bellow this line

+--------------------------------------------------------------------*/

#include <stdio.h>

#include "1394_phy.h"

/*--------------------------------------------------------------------

|	Constant definitions bellow this line

+--------------------------------------------------------------------*/

/*** Number of Link registers and registers names ***/

#define N_LINK_REG          12      // Number of Link Register

#define NODE_IDS_REG        0       // bus_id and phy_id of the node

#define RESET_START_REG     1       // a write to this register performs a reset of the bus

#define SPLIT_TIMEOUT_REG   2       // split subaction maximum timeout

#define CYCLE_TIME_REG      3       // value given by the last cycle start packet

#define BUS_TIME_REG        4       // measure of the current time in second

#define BUSY_TIMEOUT_REG    5       // maximum number of retry for a failed async transfer

#define BUS_MANAGER_ID_REG  6       // physical_ID of the current bus manager

#define BANDWIDTH_AVAIL_REG 7       // isochronous bandwidth remaining that can be allocated

#define CHANNELS_AVAIL_REG  8       // 64 bit register indicating the allocated channels

#define TOPOLOGY_MAP_REG    9       // registration of all the self_ID packets observed

#define SPEED_MAP_REG       10      // not implemented in this simulator

#define ROM_REG             11      // contains specific information concerning the node

/*** Link Register CSR standard address ***/

#define NODE_IDS_ADD        0x0008  // bus_id and phy_id of the node

#define RESET_START_ADD     0x000C  // a write to this register performs a reset of the bus

#define SPLIT_TIMEOUT_ADD   0x0018  // split subaction maximum timeout

#define CYCLE_TIME_ADD      0x0200  // value given by the last cycle start packet

#define BUS_TIME_ADD        0x0204  // measure of the current time in second

#define BUSY_TIMEOUT_ADD    0x0210  // maximum number of retry for a failed async transfer

#define BUS_MANAGER_ID_ADD  0x021C  // physical_ID of the current bus manager

#define BANDWIDTH_AVAIL_ADD 0x0220  // isochronous bandwidth remaining that can be allocated

#define CHANNELS_AVAIL_ADD  0x0224  // 64 bit register indicating the allocated channels

#define TOPOLOGY_MAP_ADD    0x0800  // registration of all the self_ID packets observed

#define SPEED_MAP_ADD       0x1000  // not implemented in this simulator

#define ROM_ADD             0x1200  // contains specific information concerning the node

// this address should be checked!!!

/*** maximum number of entries per FIFO ***/

#define FIFO_MAX_ENTRIES      16

/*--------------------------------------------------------------------

|  Type definitions bellow this line

+--------------------------------------------------------------------*/

/*** Asynchronous Header ***/

typedef struct {

short   destination_id; // destination node id of the packet

char    tl_rt;          // tl[7..2], rt[1..0]

char    tcode_pri;      // transaction_code[7..4], pri[3..0]

short   source_id;      // source node id ( bus_id, phy_id ) of the packet

short   psi[5];         // 80 bits packet specific information

long    crc;            // crc of the header field

} Async_Header; // Asynchronous Packet Header Field

/*** Asynchronous Data Block ***/

typedef struct {

short   size;       // size of the data field in byte

long    *block;     // 32 bits data block

long    crc;        // crc of the data field

} Async_Data;   // Asynchronous Packet Data Field

/*** Asynchronous Packet ***/

typedef struct {

Async_Header    header;     // header of the asynchronous data packet

Async_Data      data;       // data field of the asynchronous data packet

} Async_Packet; // Asynchronous Packet

/*** Isochronous Packet ***/

typedef struct {

short   data_size;      // size of the data field in byte

char    tag_channel;    // tag[7..6], channel[5..0]

char    tcode_sy;       // transaction_code[7..4], pri[3..0]

long    header_crc;     // crc of the header

long    *data_field;    // data field of the packet, quadlet aligned.

long    data_crc;       // crc of the data field

} Isoc_Packet;  // Isochronous Packet

/*** Link Register Type ***/

typedef struct {

short   size;       // size of the register in byte

short   csr_add;    // CSR address of the register

long    *value;     // value field of the register quadlet aligned

} Link_Reg;     // Link Register

/*--------------------------------------------------------------------

|  Class definitions bellow this line

+--------------------------------------------------------------------*/

class C1394_Link

{

protected :   // private attributes bellow this line

int             m_arf_pos;      // Asynchronous receive FIFO position ( last packet pos + 1)

int             m_atf_pos;      // Asynchronous transmit FIFO position ( last packet pos + 1)

int             m_irf_pos;      // Isochronous receive FIFO position ( last packet pos + 1)

int             m_itf_pos;      // Isochronous transmit FIFO position ( last packet pos + 1)

int             m_logical_id;   // logical id of this node

unsigned long   m_idle_time;    // link idle time;

PL_Interface    m_pl_int;       // Phy - Link Interface

Link_Reg        m_reg[N_LINK_REG];  // Link layer register

C1394_Phy       *m_phy;             // Phy layer object

Async_Packet    *m_ar_fifo[FIFO_MAX_ENTRIES];   // Asynchronous Receive FIFO

Async_Packet    *m_at_fifo[FIFO_MAX_ENTRIES];   // Asynchronous Transmit FIFO

Isoc_Packet     *m_ir_fifo[FIFO_MAX_ENTRIES];   // Isochronous Receive FIFO

Isoc_Packet     *m_it_fifo[FIFO_MAX_ENTRIES];   // Isochronous Transmit FIFO

public :    // public attributes bellow this line

// public constructor

C1394_Link( int logical_id, C1394_Phy *phy_obj = NULL );

// public virtual destructor

virtual ~C1394_Link();

// resets the link layer

void reset( void );

// indicates if the ar_fifo is empty

inline bool arf_empty( void ) { return ( m_arf_pos == 0 ) ? true : false; }

// indicates if the ir_fifo is empty

inline bool irf_empty( void ) { return ( m_irf_pos == 0 ) ? true : false; }

// sends an asynchronous packet to the at_fifo

// the return value indicates if the operation was successful

bool send_ap( Async_Packet *ap );

// sends an isochronous packet to the it_fifo

// the return value indicates if the operation was successful

bool send_ip( Isoc_Packet *ip );

// retrieves an asynchronous packet from the ar_fifo

// returns null if the ar_fifo is empty

Async_Packet *retrieve_ap( void );

// retrieves an isochronous packet from the ir_fifo

// returns null if the ir_fifo is empty

Isoc_Packet *retrieve_ip( void );

// access a link register at address add

// returns the reg value

Link_Reg acess_link_reg( bool rnw, int add );

// access a csr register at offset off

// returns the reg value

Link_Reg acess_csr_reg( bool rnw, int off );

// simulates 10ns of the link layer activity

// reports the important event happening in link to the event_log

void simulate_10ns( unsigned long date, FILE *event_log, TP *port );

};

#endif // _1394_LINK_H_

## IV. 1394_Trans.h :

/*--------------------------------------------------------------------

|

|      1394 Transaction layer header file

|      by ralao ( ENST 1998 )

|

+--------------------------------------------------------------------*/

#ifndef _1394_TRANS_H_

#define _1394_TRANS_H_

/*--------------------------------------------------------------------

|	Include definitions bellow this line

+--------------------------------------------------------------------*/

#include <stdio.h>

#include "1394_link.h"

/*--------------------------------------------------------------------

|	Constant definitions bellow this line

+--------------------------------------------------------------------*/

#define LOG_BUF_SIZE    2048    // Log buffer size

/*--------------------------------------------------------------------

|  Type definitions bellow this line

+--------------------------------------------------------------------*/

/*** Transaction Type ***/

typedef enum {

TT_UNIFIED,     // Unified transaction type, only read operation can be unified

TT_SPLIT,       // Split transaction type

TT_CONCATENATED,// Concatenated transaction type, not supported in this implementation

TT_BROADCAST    // Broadcast transaction type

} Trans_Type;       // Transaction Type

/*--------------------------------------------------------------------

|  Class definitions bellow this line

+--------------------------------------------------------------------*/

class C1394_Trans

{

protected :   // private attributes bellow this line

bool            m_lt_pending;   // indicates if a lock transaction is pending

int             m_logical_id;   // logical id of this node

int             m_rtaf_pos;     // Read Transaction Answers FIFO position ( last packet pos + 1)

int             m_lb_pos;       // log buffer position

C1394_Link      m_link;         // link object used to access the 1394 bus

Async_Packet    *m_rta_fifo[FIFO_MAX_ENTRIES];  // Read Transaction Answers FIFO

Async_Packet    *m_lt_packet;                   // Lock Transaction Packet

char            *m_log_buf[LOG_BUF_SIZE];       // Log Buffer

public :    // public attributes bellow this line

// public constructor

C1394_Trans( int logical_id, C1394_Link *link_obj = NULL );

// public virtual destructor

virtual ~C1394_Trans();

// resets the transaction layer

void reset( void );

// starts a read transaction

// returns a bool indicating if the transaction started successfully

bool read( Trans_Type type, short target_node_id );

// starts a write transaction

// returns a bool indicating if the transaction started successfully

bool write( Trans_Type type, short target_node_id, Async_Packet *packet );

// starts a lock transaction

// returns a bool indicating if the transaction started successfully

bool lock( Trans_Type type, short target_node_id, Async_Packet *packet );

// indicates if a lock transaction is pending

inline bool lt_pending( void ) { return m_lt_pending; };

// returns the lock transaction packet

inline Async_Packet *retrieve_lt_packet( void ) { return m_lt_packet; }

// answers a lock transaction

bool answer_lt( short target_node_id, Async_Packet *packet );

// indicates if the rta_fifo is empty

inline bool rtaf_empty( void ) { return ( m_rtaf_pos == 0 ) ? true : false; }

// retrieves an answer packet from the rta_fifo

// returns null if the rta_fifo is empty

Async_Packet *retrieve_rta( void );

// simulates 10ns of the transaction layer activity

// reports the important event happening in trans to the event_log

void simulate_10ns( unsigned long date, FILE *event_log );

};

#endif // _1394_TRANS_H_

## V. 1394_Controller.h :

/*--------------------------------------------------------------------

|

|      1394 Controller header file

|      by ralao ( ENST 1998 )

|

+--------------------------------------------------------------------*/

#ifndef _1394_CONTROLLER_H_

#define _1394_CONTROLLER_H_

/*--------------------------------------------------------------------

|	Include definitions bellow this line

+--------------------------------------------------------------------*/

#include <stdio.h>

#include "1394_phy.h"

#include "1394_link.h"

#include "1394_trans.h"

/*--------------------------------------------------------------------

|	Constant definitions bellow this line

+--------------------------------------------------------------------*/

/*** capabilities of the controller ***/

#define ISOC_MANAGER_CAP    0x01    // Isochronous Manager Capable

#define BUS_MANAGER_CAP     0x02    // Bus Manager Capable

#define CYCLE_MASTER_CAP    0x04    // Cycle Master Capable

/*--------------------------------------------------------------------

|  Type definitions bellow this line

+--------------------------------------------------------------------*/

/*--------------------------------------------------------------------

|  Class definitions bellow this line

+--------------------------------------------------------------------*/

class C1394_Controller

{

protected :   // private attributes bellow this line

char        m_cap;          // capabilities of this node ( Isoc manager cap, etc. )

int         m_logical_id;   // logical id of this controller

int         m_n_port;       // number of port of this controller

long        m_nrt_max;      // maximum number of data quadlet read transaction per second

long        m_nwt_max;      // maximum number of data quadlet write transaction per second

long        m_nrt_cur;      // current number of data quadlet read transaction per second

long        m_nwt_cur;      // current number of data quadlet write transaction per second

long        m_isoc_bw;      // isochronous traffic generated by this node

C1394_Phy   *m_phy;         // Phy object of this controller

C1394_Link  *m_link;        // Link object of this controller

C1394_Link  *m_trans;       // Link object of this controller

public :    // public attributes bellow this line

// public constructor

C1394_Controller(   int logical_id, char capabilities, long nrt_max,

long nwt_max, long isoc_bw = 0, int n_port = 1  );

// public virtual destructor

virtual ~C1394_Controller();

// resets the controller layer

void reset( void );

// simulates 10ns of the controller activity

// reports the important event happening in the controller to the event_log

void simulate_10ns( unsigned long date, FILE *event_log );

};

#endif	//	_1394_CONTROLLER_H_

## VI. 1394_Network.h :

/*--------------------------------------------------------------------

|

|      1394 Network header file

|      by ralao ( ENST 1998 )

|

+--------------------------------------------------------------------*/

#ifndef _1394_NETWORK_H_

#define _1394_NETWORK_H_

/*--------------------------------------------------------------------

|	Include definitions bellow this line

+--------------------------------------------------------------------*/

#include <stdio.h>

#include "1394_controller.h"

/*--------------------------------------------------------------------

|	Constant definitions bellow this line

+--------------------------------------------------------------------*/

/*--------------------------------------------------------------------

|  Type definitions bellow this line

+--------------------------------------------------------------------*/

/*** Simulation Monitor ***/

typedef struct {

bool    running;        // indicates that the simulation is running

bool    done;           // indicates that the simulation is done

bool    canceled;       // indicates that the simulation has been canceled

char    status[256];    // simulation status

} Simu_Monitor;

/*** Connection Cable ***/

typedef struct {

int lid_A;  // Logical ID of node A

int pn_A;   // Port number to use on node A

int lid_B;  // Logical ID of node B

int pn_B;   // Port number to use on node B

} CC;   // Connection Cable

/*--------------------------------------------------------------------

|  Class definitions bellow this line

+--------------------------------------------------------------------*/

class C1394_Network

{

protected :   // private attributes bellow this line

int                 m_n_controllers;// number of controllers on the bus

FILE                *m_cfg_file;    // configuration file of the network

FILE                *event_log;     // Event log file of the network

TP                  **m_port;       // array of array of Transmission pairs

CC                  *m_cc;          // array of connection cable

C1394_Controller    *m_controller;  // array of controller object

public :    // public attributes bellow this line

// public constructor

C1394_Network( const char *config_filename );

// public virtual destructor

virtual ~C1394_Network();

// resets the network

void reset( void );

// simulates the network activity for length ns

// reports the important event happening on the network to the event_log

void simulate( unsigned long simu_length, Simu_Monitor *simu_mon = NULL );

};

#endif  // _1394_NETWORK_H_

# References

- The reader who wants to know more about the IEEE 1394 bus may refer to 	the following references that were used in the elaboration of this report.

- IEEE 1394-1995 High Performance Serial Bus :  Michael D. Johas Teener, Chief Technical Officer, Firefly Inc. ( ftp://ftp.austin.ibm.com/pub/chrptech/1394ohci/1394ovw.PDF )

- Le DAWN Station de travail audio : David LELOUVIER, Ecole Nationale Supérieure des Télécommunications.

- IEEE 1394-1995 Overview : Scott Smyers, Director, Sony US Research Laboratories. ( ftp://ftp.austin.ibm.com/pub/chrptech/1394ohci/ovw2.pdf  )

- 1394 Specifications : ( http://www.firewire.org/abouttech/specifications/techspec.html )

- TSB11LV01 1-Port IEEE 1394-1995 Cable Transceiver / Arbiter : Texas Instrument, May 1997, (http://www-s.ti.com/sc/psheets/slls232b/slls232b.pdf ) .

- TSB12C01AM IEEE 1394-1995 High Speed Serial-Bus Link Layer Controller : Texas Instrument, March 1996, (http://www-s.ti.com/sc/psheets/slls219a/slls219a.pdf ) .

- An electronic copy of this report will be available at http://www.venus.org/1394 .

Unsupervised is equivalent to Plug and Play, meaning that no manual device configuration is needed.

The cable topology of the IEEE 1394 is a non cyclic network, not a bus. But the logical topology ( the one emulated by the IEEE 1394 protocol ) is a bus.