Rachad ALAO ( ralao@email.enst.fr )								06/08/98

IEEE 1394 Phy Layer Specifications

Abstract :  The goal of this paper is to describe  the specifications of an architecture of a generic Physical Layer for IEEE 1394 buses. The Document will describe the block level architecture, the internal registers available and the different states of the Phy state machine.

I . Block Level Architecture :

The Block Level architecture of the IEEE 1394 Phy layer described in the IEEE 1394 controller architecture is as follow ( see figure 1 ) :

figure 1.

- Specs of the Cable Analog Interface :

This block is responsible of the analog conversion of the logical signals generated by Phy to differential voltage level required in IEEE 1394 protocol.  The Block Interface is described in figure 2.

figure 2.

TPA , TPB :  These are the 2 differential pairs as described in IEEE 1394 standard. The data are encoded on theses pair with the following voltage level :

| Logical Value : | Voltage : |
| --- | --- |
| 0 | > 168 mV |
| Z | Between -89 mV and 89 mV |
| 1 | < -168 mV |

Power : This is the power cable as defined in IEEE 1394 standard, the available voltage on this cable is between 8 and 40 V DC, and no more than 1.5 A can circulate on one link.

A_out :  This is a 3 states input pin  ( 0, 1, Z ) that represents the value to be output on TPA.

A_in : This is a 3 states output pin ( 0, 1, Z ) that represents the value read on TPA.

B_out :  This is a 3 states input pin  ( 0, 1, Z ) that represents the value to be output on TPB.

B_in : This is a 3 states output pin ( 0, 1, Z ) that represents the value read on TPB.

- Specs of the Transmit Encoder Block :

This block knows how to generate the data and the strobe signals to be sent over the IEEE 1394 bus taking in input the data to be sent as well as the local clock. It also knows how to encode specific status code on TPA and TPB. The Block Interface is described in figure 3.

figure 3.

A_out :  This is a 3 states output pin  ( 0, 1, Z ) that represents the value to be output on TPA.

B_out :  This is a 3 states output pin  ( 0, 1, Z ) that represents the value to be output on TPB.

T_Data : These inputs represent the data to be transmitted on the bus if Data/Status pin is high. They are encoded as follow :

00  0, 01  Z, 10  1, 11  Invalid

Trans_clk :  The transmission clock input pin is the clock that will be used to send the data to the bus. This clock is active high on rising edge.

Data / Status :  This input pin indicates whether we are transmitting data ( Data/Status =  High ) or we are transmitting status ( Data/Status = Low ).

T_Status : These inputs represents the status to be transmitted  to the bus if Data/Status pin is low. Status are in format following bellow:

| T_Status ( Hex ) | Status | TPA, TPB values |
| --- | --- | --- |
| 0 | IDLE | Z, Z |
| 1 | TX_REQ | Z, 0 |
| 2 | TX_GRANT | Z, 0 |
| 3 | TX_PARENT_NOTIFY | 0, Z |
| 4 | TX_DATA_PREFIX | 0, 1 |
| 5 | TX_CHILD_NOTIFY | 1, Z |
| 6 | TX_IDENT_DONE | 1, Z |
| 7 | TX_DATA_END | 1, 0 |
| 8 | BUS_RESET | 1, 1 |
| 9 | Reserved | - |
| … | … | … |
| F | Reserved | - |

- Specs of the Receive Decoder Block :

This block knows how to decode the data and the strobe on the bus. It also knows how to interpret the various states of the bus and knows which status code to output according to these states. The Block Interface is described in figure 4.

figure 4.

A_in : This is a 3 states input pin ( 0, 1, Z ) that represents the value read on TPA.

A_out :  This is a 3 states input pin  ( 0, 1, Z ) that represents the value to be output on TPA.

B_in : This is a 3 states input pin ( 0, 1, Z ) that represents the value read on TPB.

B_out :  This is a 3 states input pin  ( 0, 1, Z ) that represents the value to be output on TPB.

R_Data : These outputs represent the data received from the bus if Data/Status pin is high. They are encoded as follow :

00  0, 01  Z, 10  1, 11  Invalid

Recv_clk :  The reception clock output pin is the clock that was used to send the data on the bus. This clock is active high on rising edge.

Data / Status :  This input pin indicates whether we are receiving data ( Data/Status =  High ) or we are receiving status ( Data/Status = Low ).

R_Status : These outputs represents the status received from the bus if Data/Status pin is low. Status are in format following bellow:

| R_Status ( Hex ) | Status | TPA, TPB values |
| --- | --- | --- |
| 0 | IDLE | Z, Z |
| 1 | RX_PARENT_NOTIFY | Z, 0 |
| 2 | RX_REQ_CANCEL | Z, 0 |
| 3 | RX_IDENT_DONE | Z, 1 |
| 4 | RX_SELF_ID_GRANT | 0, Z |
| 5 | RX_REQUEST | 0, Z |
| 6 | RX_ROOT_CONTENT | 0, 0 |
| 7 | RX_GRANT | 0, 0 |
| 8 | RX_PARENT_HDSHK | 0, 1 |
| 9 | RX_DATA_END | 0, 1 |
| A | RX_CHILD_HDSHK | 1, Z |
| B | RX_DATA_PREFIX | 1, 0 |
| C | BUS_RESET | 1, 1 |
| D | Reserved | - |
| E | Reserved | - |
| F | Reserved | - |

- Specs of the Phy State Machine :

This Block is the heart of the Phy layer. It is a state machine that knows how to arbitrate the bus on reset or for transmission purpose. It knows how to communicate with the Link layer Block via the Phy / Link bus.  The Block Interface is described in figure 5.

figure 5.

T_Data : These outputs represent the data to be transmitted to the bus if Data/Status pin is high. They are encoded as follow :

00  0, 01  Z, 10  1, 11  Invalid

T_clk :  The transmission clock output pin is the clock that will be used to send the data on the bus. This clock is active high on rising edge.

R_Data : These inputs represent the data received from the bus if Data/Status pin is high. They are encoded as follow :

00  0, 01  Z, 10  1, 11  Invalid

R_clk :  The reception clock output pin is the clock that was used to send the data on the bus. This clock is active high on rising edge.

Data / Status :  This output pin indicates whether we are transmitting / receiving data ( Data/Status =  High ) or we are transmitting / receiving status ( Data/Status = Low ).

T_Status : These outputs represents the status to be transmitted  to the bus if Data/Status pin is low.

R_Status : These inputs represents the status received from the bus if Data/Status pin is low.

Receive : This output indicates the Link layer that data are being received from the bus.

Transmit_Granted :  This output indicates the Link layer that a previous transmit request has been granted.

Hold : This input indicates the Phy layer that the Link Layer is preparing the data to be sent after the bus was granted and that the Phy layer should try to hold the bus as long as possible.

Link_req : This input indicates the Phy layer that the bus layer want to send data on the bus. A bus request must be sent in the command register of the Phy layer concurrently to the activation of this line.

Link_DS : This input indicates that the data on Link_data  and the address on Add are strobe and can be sampled by the Phy Layer.

R/W :  This input indicates whether the Link want to execute a read ( R/W = High ) or a Write ( R/W = Low ) operation.

Add : This 8 bits input specifies the address where the next read or write operation should take place.

Link_Data :  This 8 bits input represents the data to be written in the Phy Layer by the Link layer.

Phy_DS : This output indicates that the data on Phy_Data are strobe and can be  sampled by the Link layer.

Phy_Data : This 8 bits output represents the data to be written in the Link Layer by the Phy Layer.

Clk : This is the Phy/Link clock, it is a 49.152 Mhz Clock as defined in the IEEE 1394 standard.

Link_On : This output indicates the Link Layer that the Link is on the IEEE 1394 cable.

Power_Down : This input indicates the Phy Layer that the device is off.

/Reset :  This input is the general reset input. It will reset the Phy layer and may resets the bus.

The Phy State machine contains internal registers that can be accessed by the Link layer. Here follows the description of these registers :

| Access | Address( Hexa ) | Register name and Topology |
| --- | --- | --- |
| R | 00 | T/R_Status = ¦¦ T_Status[7..4] ¦¦ R_Status[3..0] ¦¦ |
| R | 01 | Node_Conf = ¦¦ Child(7) ¦¦ Root(6) ¦¦ Phy_Id[5..0] ¦¦ |
| R | 02 | Cable_Port_Status = ¦¦ Cable_On(7) ¦¦ Cable_Power_Status(6) ¦¦ N_port[5..2] ¦¦ Speed_Max[1..0] ¦¦ |
| R/W | 03 | Gap_Count_Register = ¦¦ Gap_Counter[7..0] ¦¦ |
| R/W | 04 | Interrupt_Reg = ¦¦ Cable_Power_Low(7) ¦¦ Loop_Timeout(6) ¦¦ Reserved[5..0] ¦¦ |
| R/W | 05 | Reset_Management_Reg = ¦¦ Initiate_Bus_Reset(7) ¦¦ Reset_Initiated(6) ¦¦ Root_Hold_Off(5) ¦¦ Reserved[4..0] ¦¦ |
| R/W | 06 | Command Register = ¦¦ Com_Reg[7..0] ¦¦ |
| R/W | 07 | Transmit_Data_Reg = ¦¦ TX_DATA[7..0] ¦¦ |
| R | 08 | Phy Version = ¦¦ version[7..0] ¦¦ |

The Phy State Machine states are not described in this paper as they are too close from implementation level. Details on this state machine may be added latter in this paper, otherwise please refer to the phy state machine VHDL code.